Attention-based Non-stationary Gaussian Process Modeling and Robust Parameter Optimization

Abstract:

To address the challenge of robust parameter optimization for non-stationary responses under small-sample conditions, this study proposes a robust optimization method within the non-stationary Gaussian process (NSGP) framework, incorporating a two-stage attention-weighting mechanism. The method first establishes a primary attention stage by introducing a quality loss function, which prioritizes experimental points that enhance the modeling quality of the response surface. A secondary attention stage is then developed by integrating a maximin distance space-filling (MDSF) strategy to select points that improve spatial distribution efficiency. Subsequently, a robust parameter optimization model is formulated based on the predicted response and its associated uncertainty derived from the NSGP model. Finally, a Genetic Algorithm (GA) is employed to search the feasible input domain for the optimal parameter combination. Simulation results demonstrate that, in limited-sample settings, the proposed method significantly improves predictive accuracy within the optimal solution subspace and enhances the overall performance of the parameter optimization model, thereby increasing the robustness and reliability of the final solution.

Keywords: Attention Mechanism; Robust Parameter Design; Non-stationary Response; Gaussian Process Modeling; Small-sample Data

# 1 Introduction

In modern manufacturing, product quality is the cornerstone of corporate competitiveness and customer loyalty, playing a pivotal role in ensuring customer satisfaction and enhancing market position[1]-[2]. Superior product quality not only elevates brand value but also mitigates costs associated with defects, rework, and warranty services. Consequently, quality improvement has become a central and persistent focus within the global manufacturing sector[3]. Robust Parameter Design (RPD), a key technology in quality engineering, aims to optimize product and process parameters to reduce the sensitivity of system performance to noise factors, thereby enhancing product stability and consistency[4]. This methodology has found widespread application in high-end manufacturing, engineering design, and service industries, providing a solid theoretical and practical foundation for improving the performance of complex systems in uncertain environments. While Response Surface Methodology (RSM) is a conventional approach for implementing RPD by modeling the relationship between input variables and output responses, traditional RSM models have proven inadequate in fitting accuracy and generalization capability when applied to increasingly complex and intelligent industrial systems.

In recent years, Gaussian Process (GP) models have gained significant traction in machine learning due to their exceptional non-parametric Bayesian modeling capabilities. They have been successfully applied to regression, uncertainty quantification, and optimization design, demonstrating marked advantages in handling high-dimensional, non-linear problems[4]-[6]. Numerous researchers have integrated GP models into RPD. For instance, Tan and Wu[7] proposed a quadratic expected quality loss function based on a GP model to improve the accuracy of robust optimal solutions. Sun et al.[8] introduced a rapid, pre-evaluation-based GP modeling method to enhance predictive accuracy. Bogunovic et al.[9] developed a robust GP optimization method based on confidence bounds to improve decision-making reliability under uncertainty. Furthermore, Zhang et al.[10] utilized GP models to enhance robustness in semiconductor manufacturing and tunnel engineering. These studies have significantly advanced the theory of GP-based robust optimization. However, a fundamental assumption of the classic GP model is process stationarity, which posits a constant covariance function across the entire input space. This assumption limits its efficacy in modeling non-stationary responses characterized by sharp local variations and multi-scale features—phenomena prevalent in frontier engineering fields such as aerospace and additive manufacturing[11]-[13].

To overcome the limitation of the stationarity assumption, Non-stationary Gaussian Process (NSGP) models have been developed. The treed partitioning NSGP model proposed by Gramacy and Lee[14] successfully simulated non-stationary phenomena in rocket booster design. Konomi et al.[15] extended the NSGP framework to multi-response systems to model complex fluid functions in carbon capture processes. Pandita et al.[16] constructed a latent response function using NSGP to improve the robustness of a specific manufacturing process, while Paun et al.[17] developed a variational inference-based NSGP model to enhance scalability for large datasets. Although these studies confirm the potential of NSGP for characterizing complex response surfaces, NSGP models typically possess more intricate structures and a greater number of hyperparameters. In limited-data, or small-sample, scenarios, model construction is highly susceptible to overfitting or underfitting, which degrades predictive accuracy and severely compromises the validity and reliability of subsequent robust parameter optimization. Therefore, developing methods for the efficient and accurate construction of NSGP models under small-sample conditions has become a critical challenge in the field of robust parameter design.

To address the challenge of accurately modeling non-stationary responses from small datasets, researchers have explored active learning-based GP methods. Antunes et al.[18] proposed an active learning-based meta-modeling approach to improve input space exploration and reduce model output uncertainty. Meng et al.[19] developed a hybrid GP model and employed an active learning algorithm to mitigate the computational burden of repeated structural analyses. Wang and Broccardo[20] introduced an active learning-based GP metamodel to provide accurate estimates for median low-probability regions. Similarly, Han et al.[21] proposed an efficient active learning algorithm for GP models to estimate response contours from computationally expensive models while accounting for input noise. Concurrently, the rise of deep learning has introduced the attention mechanism as a novel paradigm for this problem. Inspired by the human cognitive ability to focus on salient information[22], the attention mechanism adaptively assigns weights to input features, enabling a model to concentrate its resources on the most critical information for a given task. In regression, this allows the model to identify and prioritize regions or dimensions of the input space that most significantly influence the output. This characteristic aligns perfectly with the requirements of NSGP modeling, where different regions of a non-stationary response surface exhibit varying levels of complexity and contribute differently to global performance. By incorporating an attention mechanism, a model can intelligently identify "key regions"—areas of rapid functional change or high impact on robustness metrics—and allocate greater modeling focus to them, thereby enhancing both local fitting accuracy and overall predictive performance with limited data. For example, the Transformer architecture, proposed by Vaswani et al.[23], relies entirely on self-attention to capture global dependencies, drastically improving parallelization and reducing training time. Li et al.[24] re-envisioned the mixture of Gaussian processes (MGP) model through an attention-based lens, replacing probabilistic weights with more flexible, distance-based attention weights to extend the MGP framework to more complex data structures. Xie et al.[25] used a GP as a prior to explicitly model inter-channel correlations, generating more interpretable and performant attention weights. Zhong et al.[26] constructed a Gaussian Attention Module (GAM) that uses a self-attention-like mechanism to capture global context and a Gaussian function to re-weight weak signals from ambiguous boundaries, solving challenges in wood surface defect segmentation. Huynh et al.[27] combined attention, GPs, and weak supervision to effectively address the scarcity of labeled data in medical AI. These studies demonstrate the viability of integrating attention mechanisms with Gaussian processes and suggest their potential for enabling accurate NSGP modeling from small samples.

Building on this analysis, this study addresses the problem of robust parameter design for non-stationary responses from small-sample data by proposing a novel modeling and optimization method that integrates a two-stage attention-weighted NSGP. The methodology unfolds as follows: first, a foundational NSGP model is constructed to provide an initial characterization of the response's non-stationary properties. Second, a two-stage attention mechanism is introduced. The primary stage, guided by a quality loss function, assigns higher weights to experimental points that improve the response surface model's quality. The secondary stage then selects a subset of these points, integrating a maximin distance space-filling (MDSF) strategy to prioritize points that enhance spatial distribution efficiency. Subsequently, a more precise robust parameter optimization model is formulated based on this attention-enhanced NSGP. A Genetic Algorithm (GA) is then employed to solve for the optimal parameter set. Finally, the efficacy of the proposed method is validated through numerical simulations and an engineering case study. The primary contribution of this work, compared to existing NSGP methods, is the use of an attention mechanism to augment the model's information extraction and feature learning capabilities in a small-sample context. This approach particularly strengthens the characterization of critical local features within the non-stationary response, namely those in the potential sub-region of the optimal solution. This enables the model to leverage valuable experimental data more efficiently, leading to significant improvements in predictive accuracy and providing a robust foundation for obtaining more reliable optimal solutions. The overall workflow is illustrated in Figure 1.

Figure 1. Workflow of the two-stage attention-weighting mechanism.

# 2 Gaussian Process Model

## 2.1. GP Modeling Processes

A Gaussian Process (GP) offers a non-parametric Bayesian framework for inference over functions. A central feature of this approach is that instead of yielding a deterministic value , it defines a full Gaussian probability distribution over the function's value at any given input . This property enables a GP not only to provide point predictions but also to quantify the associated uncertainty, typically expressed as a confidence interval. The modeling adheres to the Bayesian paradigm: a GP prior is first specified for the latent function, and this prior is subsequently updated with observed data to yield a posterior probability distribution that characterizes the function.

Within the GP framework, the prior distribution over the function values corresponding to any finite set of input points,, is defined by a mean function and a covariance function. The mean function is given by:

whereis a vector of basis functions evaluated at the input , and is the corresponding vector of regression coefficients. To simplify the model structure, the mean function is often set to zero or defined by a simple linear form, such as .

The covariance structure is specified by a covariance function, or kernel,..,which quantifies the correlation between the function values at any two inputs,and :

The kernel is governed by a set of hyperparameters, including the signal variance (a scaling factor), the noise variance , and , a diagonal matrix whose elements are the characteristic length-scales for each input dimension. The length-scales describe the smoothness of the function along each respective dimension. These hyperparameters are typically learned from data by optimizing a criterion such as the marginal likelihood.

For the widely used Squared Exponential (SE) kernel, Equation (2) takes the form:

In Equation (3),is the Kronecker delta, which equals 1 if and 0 otherwise. This term models observation noise and ensures the positive definiteness of the resulting covariance matrix. If the kernel is assumed to be isotropic, implying uniform smoothness across all input dimensions, the length-scale matrix can be parameterized by a single scalar . The SE kernel then simplifies to its isotropic form:

This completes the specification of the prior distribution over the function. Posterior inference in a GP model is conditioned on a set of training data . A fundamental property of a GP is that the joint distribution of function values at any finite collection of points is a multivariate Gaussian. Leveraging the closure properties of the Gaussian distribution, the analysis can be extended to a finite set of points comprising both training inputs and new test inputs . Let  and  be the latent function values at and , respectively. Under a zero-mean assumption, their joint prior distribution is:

where is the covariance matrix evaluated at all pairs of training points,is the vector of covariances between the training points and the test point, and is its transpose.The relationship between the observed data and the latent function values is typically modeled by introducing Gaussian noise:

where is the identity matrix and  is the variance of the observation noise. To obtain the distribution over the observed values, known as the marginal likelihood, the latent variables are integrated out of the joint distribution:

Due to the properties of Gaussian distributions, this integral has an analytical solution. Consequently, the joint distribution of the observed training data and the predictive output at a test location is also Gaussian:

By applying the formula for the conditional distribution of a multivariate Gaussian, the posterior predictive distribution for , given the training data, can be derived analytically from this joint distribution:

The posterior mean and variance are given by:

The posterior mean serves as the model's point prediction, while the posterior variance quantifies the associated uncertainty and is used to construct confidence intervals.

The model's hyperparameters, collectively denoted as , are not fixed a priori but are instead optimized in a data-driven manner. The standard approach is to maximize the log marginal likelihood (LML):

where is the number of training samples.This objective function inherently balances model fit (the first term) against model complexity (the second term, a log-determinant penalty). Optimal estimates for are obtained by maximizing this function, typically using gradient-based optimization algorithms.

## 2.2  Non-stationary Gaussian Processes

In a conventional Gaussian process, the spatial correlation structure is governed by the length-scale parameter, l, of the covariance function. Standard implementations typically employ an isotropic assumption, applying a single, uniform length-scale across all input dimensions. This assumption implies that the function's smoothness or rate of change is constant throughout the feature space. However, this prior becomes inappropriate when the target function exhibits non-stationary characteristics. Non-stationary or piecewise response functions often display sharp, short-scale variations in some regions and smooth, long-scale trends in others. A single, globally shared length-scale cannot accommodate such heterogeneous behavior, resulting in a model that fails to capture both local details and the global structure accurately.

To effectively model this heterogeneity, a more adaptive covariance structure is required. This study employs a regression tree-based Gaussian process model. The core mechanism of this approach is the recursive partitioning of the input space into multiple sub-regions. This partitioning aims to create sub-regions where the function's behavior is locally stationary. Subsequently, independent, local GP models are fitted within each of these sub-regions. This intelligent partitioning of the input space constitutes the framework for a non-stationary Gaussian process (NSGP), with the detailed tree-based modeling procedure provided in Appendix A. The posterior predictive distribution of the model is derived using linear least squares, resulting in a Student's t-distribution:

A key distinction of this model is that the posterior predictive distribution for the latent function follows a Student's t-distribution, a significant departure from the Gaussian distribution (Eq. (9)) of a standard GP. This form arises from the Bayesian framework; marginalizing the process variance, hyperparameters, and regression coefficients () results in a Student's t-distribution, which has heavier tails than a Gaussian. The distribution has degrees of freedom, where  is the number of training samples and is the number of free parameters. As the degrees of freedom increase, this distribution approaches a Gaussian, whereas for small degrees of freedom, its heavy tails provide a more robust estimate of predictive uncertainty.

Accordingly, the predictive mean and variance for the NSGP model are given by:

The predictive mean in Eq. (14) can be interpreted as a composite structure: an explicit regression term,, which captures the global data trend, and a GP correction term, ,which models local nonlinear deviations not accounted for by the basis functions. In Eq. (15),represents the prior variance at the prediction point. Other key components include the vector of covariances between the prediction point and the training points,, and the design matrixevaluated at the training points:

The matrix in Eqs. (14-15) is the symmetric positive-definite kernel (or Gram) matrix for the training data, with elements given by the kernel function evaluated at pairs of training points:

The regression coefficients in Eqs. (14-15) are estimated using Generalized Least Squares (GLS):

The process variance,, is estimated as a weighted sum of squared residuals:

Where, the denominator represents the effective degrees of freedom, which accounts for the parameters estimated from the data. The vector in these equations contains the observed training outputs, which, along with and , define the posterior distribution:

These components form the fundamental framework of the NSGP model. By partitioning the input space into simpler, locally stationary sub-regions and fitting a distinct GP to each, this approach effectively models multi-scale phenomena. This strategy enhances the model's capacity to capture heterogeneous behavior across the input domain, thereby improving predictive performance for non-stationary responses.

# 3 TGP Modeling with Two-Stage Attention Weighting and a Quality Loss Function

## 3.1 Attention-Based Weighting

The proposed method is a two-stage attention mechanism designed to optimize regression models. This mechanism leverages the core principle of attention networks: assigning distinct importance weights to candidate points in a data pool to strategically select the most informative sample for addition to the training set. The objective is to efficiently enhance the model's predictive accuracy in the region surrounding the optimum while maintaining good spatial coverage of the design space.

# Given a pool of candidate points,, the goal is to select a new point,, to augment the existing training set,. This selection is governed by the two-stage attention mechanism. Analogous to how Hierarchical Attention Networks (HAN) use a hidden layer to generate representations, this framework employs two metrics to evaluate the utility of each candidate point. To transform these metrics into normalized importance weights, a score derived from them is first processed through negation and logarithmic scaling. A Softmax function then computes the attention weight for each candidate point at each stage:

Where,  represents the processed score for candidate point , and the denominator is a normalization term that ensures the weights sum to unity. This combination of exponentiation and normalization effectively highlights the most valuable candidate points. The specific construction of the metrics is detailed in the following section.

## 3.2 Two-Stage Attention-Based NSGP Modeling Algorithm

The attention mechanism is a machine learning technique that directs a model's focus towards more salient parts of the input by assigning differential weights. In regression modeling, this allows an algorithm to proactively identify and select the most informative data points, thereby enhancing the efficiency of sequential experimentation. This approach is particularly effective for prediction and optimization problems in small-data regimes, such as those encountered in aerospace, nuclear engineering, and additive manufacturing.

The proposed attention-weighted algorithm comprises four primary steps:

1.Initial Model Training: An NSGP model structure is established, initial training data are generated, and the initial NSGP model is fitted.

2.Candidate Set Selection: An attention mechanism weights a quality loss function to select a subset of points with the highest weights, which form the candidate set.

3.New Experimental Point Selection: The attention mechanism is then used to weight a maximin distance criterion, selecting the single highest-weighted point from the candidate set for addition to the training data.

4.Model Update: The NSGP model is updated with the augmented training set, and the process iterates from step (2) until a stopping criterion is met.

The construction of this algorithm is detailed as follows. First, to enable the model to rapidly identify the promising subregion containing the optimum, a penalty term based on a quality loss function is designed. This term prioritizes candidate points with predicted responses close to a target value and low predictive uncertainty. The concept of quality loss, as defined in[28], posits that any deviation of a quality characteristic from its predefined target value incurs a loss, and this loss is proportional to the magnitude of the deviation.

For each candidate point , a quality loss function,, is defined to quantify its desirability:

Where  is the predicted mean response, is the target response value, and  is the predictive variance at . This function penalizes points with predictions far from the target and points with high uncertainty. To convert this penalty into an attention weight, the quality loss is passed through a Softmax function, yielding the exploitation-focused weight:

Spatial clustering of design points can degrade the global accuracy of the surrogate model and may cause the covariance matrix in a GP model to become ill-conditioned. To mitigate this, the maximin distance criterion is adopted. The space-filling score, , for a candidate point relative to the set of existing design points is its minimum Euclidean distance to any point in :

Equation (25) represents the Euclidean distance between the candidate point  and the selected point . Maximizing the spatial distance in Equation (26) can avoid local clustering of experimental points. The scoring function for the space-filling property is defined as:

Equation (26) can assign a value to the space-filling property of the candidate points, and then using the aforementioned attention mechanism, its space-filling property is attention-weighted to obtain the space-filling weight for each candidate point:

In order to consider both the optimality and the space-filling property of the design points, this paper combines Equation (24) and Equation (27) to propose a two-stage attention weighting: first, use Equation (24) to evaluate the response penalty performance of the candidate points, and use Equation (28) to select n candidate points :

Within this reduced set , the space-filling weights,, from Eq. (29) are then calculated. The point with the highest space-filling weight is selected as the new design point,:

This two-stage acquisition process ensures that the new design point  is selected from a subregion with high potential for optimality, while also promoting uniform coverage within that region. This targeted approach significantly enhances sampling efficiency. The complete methodology for Non-stationary Gaussian Process (NSGP) modeling using this attention-weighted acquisition function is detailed in Algorithm 1.

Algorithm 1 NSGP Modeling with Attention-Weighted Acquisition

Input: Initial design of experiments (DoE) dataset ; Candidate set ; Batch size ; Maximum number of iterations ;Iteration . 1. Fit an initial NSGP model using . 2. Let the current training set be . 3. while do 4. Use the current training set to fit the NSGP model parameters. 5. Predict the mean and variance for all points. 6. Calculate the response penalty weights for all points in using Eq. (24). 7. Form the reduced set by selecting the top n points with the lowest penalty weights, as per Eq. (28). 8. for do 9. Calculate the space-filling weights for all points currently in using Eq. (27). 10. Select the next point to add using Eq. (29). 11. Augment the training set:. 12. Remove the selected point from consideration:. 13. Re-fit the NSGP model with the augmented set for a fully sequential update. 14. end for 15.end Output: The final NSGP model and the augmented experimental dataset

## 3.3 Formulation of the Parameter Optimization Model for a Non-Stationary Gaussian Process with a Two-Stage Attention Mechanism

The workflow for formulating the parameter optimization model, which integrates a non-stationary Gaussian process with a two-stage attention-weighting mechanism, is illustrated in Figure 2. The procedure consists of the following steps:

Step 1: Construct the non-stationary Gaussian process (NSGP) model architecture, generate an initial training dataset, and fit the initial NSGP model.

Step 2: Formulate a quality loss function based on the output of the initial model. The weights for the different criteria are then calculated in conjunction with the maximize minimum distance (MDSF) criterion.

Step 3: Select new sampling points using the method detailed in Algorithm 1.

Step 4: Update the training dataset with the new points and refit the NSGP model.

Step 5: Establish the quality loss function, derived from the updated model's output, as the optimization objective.

Step 6: Employ a genetic algorithm (GA) to search for the global optimum within the feasible domain of the input variables, thereby identifying the optimal parameter settings.

Figure 2 Workflow for formulating the parameter optimization model for a non-stationary Gaussian process with a two-stage attention mechanism.

# 4 Case Analysis

## 4.1 Two-dimensional numerical case

This section evaluates the performance of the proposed method using a two-dimensional benchmark function. This piecewise function is designed to exhibit significant non-stationary characteristics within its two-dimensional domain. For an input point , the function is defined as:

In the region , the function comprises two components: a sinusoidal term,, multiplied by a coefficient that is linear in , and a quadratic term,. The inclusion of the sine function induces periodic oscillations along the x-axis, indicating pronounced non-stationarity in this subdomain. Conversely, in the region , the function transitions to a form based on a quadratic term,, which is modulated by an exponential correction term,. This expression exhibits circular symmetry, with function values increasing smoothly as andincrease, presenting a behavioral pattern distinctly different from that of theregion.

This transition from oscillatory to smoothly symmetric behavior at the boundary underscores the function's cross-domain heterogeneity. Such non-stationary features make it an ideal benchmark for assessing a model's capacity to adapt to complex data patterns.

For the experiment, a specific target value,, is specified. A sub-region of interest is then defined as the set of points satisfying, where is a small threshold. This set is considered the potential sub-region of the optimal solution, and the model's predictive accuracy is critically evaluated within this area. This focused approach allows for a rigorous examination of the model's performance near the optimum. For this study,is set to 1.5, a value intentionally offset from the function's extrema to mitigate potential biases.

To quantify the discrepancy between the model's predictions and the true function values, the Root Mean Square Error (RMSE) is adopted as the primary performance metric. The RMSE is defined as:

Where is the model's prediction at point ,is the true value, and nnn is the total number of evaluation points. By squaring the residuals before averaging, the RMSE is particularly sensitive to large errors, making it highly suitable for evaluating model fit on non-stationary functions. This sensitivity provides a distinct advantage in measuring a model's adaptability to regional variations in the function's behavior.

The experimental procedure begins with the selection of 20 initial samples using Latin Hypercube Sampling (LHS) to fit an initial NSGP model. Subsequently, the proposed attention-based method is employed to sequentially add 20 additional samples, yielding a final dataset of 40 samples, which is then used to update the NSGP model. For a comparative analysis, the performance of this approach is benchmarked against four established design of experiments (DoE) strategies: G-Optimality, D-Optimality[29], LHS[30], and Centroidal Voronoi Tessellations (CVT). These methods, which are classic in the field of computer experiments, are used to generate datasets of the same size (40 points). They represent four distinct sampling objectives: minimizing the maximum prediction variance (G-Optimality), maximizing the determinant of the information matrix (D-Optimality), ensuring uniform spatial coverage (LHS), and generating centroids of corresponding Voronoi regions (CVT)[31]. This comparison evaluates the quality of the response surface model guided by the proposed attention mechanism against these conventional strategies. The spatial distribution of the 40 samples generated by the proposed method is visualized in Figure 1.

Figure 3 Spatial distribution of the 40 sample points generated by the PM and LHS methods.

Table 1 RMSE results for 10 design samples selected by the attention mechanism.

| Total sample size | Mean |  |  |  |  | Median |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.0919 | 0.0894 | 0.078 | 0.0651 | 0.0562 | 0.0854 | 0.0823 | 0.0675 | 0.0639 | 0.0503 |
| 50 | 0.0621 | 0.065 | 0.0565 | 0.056 | 0.0386 | 0.0621 | 0.0663 | 0.0539 | 0.0586 | 0.0382 |
| 60 | 0.0514 | 0.054 | 0.0451 | 0.0532 | 0.0369 | 0.0487 | 0.0548 | 0.0448 | 0.043 | 0.0362 |
| 70 | 0.0427 | 0.0442 | 0.0392 | 0.0365 | 0.0317 | 0.043 | 0.0419 | 0.0349 | 0.0357 | 0.0325 |
| 80 | 0.0369 | 0.0416 | 0.033 | 0.0333 | 0.0246 | 0.0365 | 0.039 | 0.032 | 0.0309 | 0.0241 |
| 90 | 0.0308 | 0.0323 | 0.0261 | 0.0296 | 0.0216 | 0.0288 | 0.0317 | 0.0266 | 0.0264 | 0.0204 |
| 100 | 0.0225 | 0.0249 | 0.0238 | 0.0252 | 0.0181 | 0.0228 | 0.025 | 0.0228 | 0.0232 | 0.0192 |

Table 2 RMSE results for 20 design samples selected by the attention mechanism.

| Total sample size | Mean |  |  |  |  | Median |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.1517 | 0.0954 | 0.073 | 0.0817 | 0.054 | 0.1385 | 0.089 | 0.0644 | 0.0802 | 0.031 |
| 50 | 0.099 | 0.053 | 0.0538 | 0.0719 | 0.0324 | 0.0954 | 0.053 | 0.051 | 0.073 | 0.0249 |
| 60 | 0.0499 | 0.0481 | 0.0396 | 0.0565 | 0.0216 | 0.0481 | 0.0458 | 0.039 | 0.0562 | 0.0207 |
| 70 | 0.0453 | 0.041 | 0.0359 | 0.0511 | 0.0235 | 0.0466 | 0.0422 | 0.0356 | 0.0503 | 0.0222 |
| 80 | 0.0345 | 0.04 | 0.0359 | 0.0419 | 0.0223 | 0.0367 | 0.0378 | 0.0347 | 0.0431 | 0.0205 |
| 90 | 0.0319 | 0.0298 | 0.0263 | 0.0391 | 0.0187 | 0.0298 | 0.031 | 0.0263 | 0.0377 | 0.018 |
| 100 | 0.0281 | 0.0321 | 0.0227 | 0.0354 | 0.0173 | 0.0281 | 0.0328 | 0.023 | 0.0352 | 0.0164 |

（a）10 design samples selected by the attention mechanism        (b)  20 design samples selected by the attention mechanism.

Figure 4 Comparison of RMSE results within potential subregions of the optimal solution

（a）10 design samples selected by the attention mechanism        (b)  20 design samples selected by the attention mechanism.

Figure 5 Comparison of RMSE results within global response surface

Table 3 Relative performance improvement of the PM method with 10 attention-selected design samples.

| Total sample size | Mean |  |  |  | Median |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | D_opt | G_opt | LHS | CVT |
| 40 | 38.85% | 37.14% | 27.95% | 13.67% | 41.10% | 38.88% | 25.48% | 21.28% |
| 50 | 37.84% | 40.62% | 31.68% | 31.07% | 38.49% | 42.38% | 29.13% | 34.81% |
| 60 | 28.21% | 31.67% | 18.18% | 30.64% | 25.67% | 33.94% | 19.20% | 15.81% |
| 70 | 25.76% | 28.28% | 19.13% | 13.15% | 24.42% | 22.43% | 6.88% | 8.96% |
| 80 | 33.33% | 40.87% | 25.45% | 26.13% | 33.97% | 38.21% | 24.69% | 22.01% |
| 90 | 29.87% | 33.13% | 17.24% | 27.03% | 29.17% | 35.65% | 23.31% | 22.73% |
| 100 | 19.56% | 27.31% | 23.95% | 28.17% | 15.79% | 23.20% | 15.79% | 17.24% |

Table 4 Relative performance improvement of the PM method with 20 attention-selected design samples.

| Total sample size | Mean |  |  |  | Median |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | D_opt | G_opt | LHS | CVT |
| 40 | 64.40% | 43.40% | 26.03% | 33.90% | 77.62% | 65.17% | 51.86% | 61.35% |
| 50 | 67.27% | 38.87% | 39.78% | 54.94% | 73.90% | 53.02% | 51.18% | 65.89% |
| 60 | 56.71% | 55.09% | 45.45% | 61.77% | 56.96% | 54.80% | 46.92% | 63.17% |
| 70 | 48.12% | 42.68% | 34.54% | 54.01% | 52.36% | 47.39% | 37.64% | 55.86% |
| 80 | 35.36% | 44.25% | 37.88% | 46.78% | 44.14% | 45.77% | 40.92% | 52.44% |
| 90 | 41.38% | 37.25% | 28.90% | 52.17% | 39.60% | 41.94% | 31.56% | 52.25% |
| 100 | 38.43% | 46.11% | 23.79% | 51.13% | 41.64% | 50.00% | 28.70% | 53.41% |

As demonstrated in Fig. 4 and Table 3, the Proposed Method (PM) consistently achieves the lowest mean Root Mean Square Error (RMSE) across all training set sizes. For all methods, the RMSE within the optimal solution subregion decreases as the number of training points increases. With a training set of 40 samples, the PM method yields a mean RMSE of 0.0562. This represents an improvement of approximately 13.67% over the next-best performing method, CVT (0.0651), and 38.85% over the worst-performing method, D-Optimality (0.0919). This competitive advantage persists as the sample size grows. At 100 training samples, the mean RMSE of the PM method drops to 0.0181, outperforming the best competitor at this stage, D-Optimality (0.0225), by 19.56%, and the worst performer, CVT (0.0252), by 28.17%. The maximum observed improvement was 40.87% over D-Optimality with 80 training samples, while the minimum was 13.15% over CVT with 70 samples.

The median RMSE, an indicator of typical performance, further underscores the superiority and robustness of the PM method. With 40 samples, the PM method's median RMSE is 0.0503. The median RMSE of the best-performing competitor, CVT (0.0639), is 21.28% higher, while that of the worst-performer, D-Optimality (0.0854), is 41.1% higher. At 100 samples, the PM method's median RMSE is 0.0192. The next-best methods, D-Optimality and LHS, both record a median RMSE of 0.0228 (15.79% higher), while the worst-performing method records 0.0250 (23.20% higher). In terms of median RMSE, the greatest improvement was 42.38% over G-Optimality with 50 samples, and the smallest was 6.88% over LHS with 70 samples.

The data in Fig. 4 and Table 4 show that when the batch size for adding new samples is increased to 20, the performance advantage of the PM method becomes more pronounced, particularly in the early learning stages with smaller training sets. With 40 total samples, the PM method achieves a mean RMSE of 0.0540, which is 26.03% lower than the best competitor, LHS (0.0730), and 64.4% lower than the worst performer, D-Optimality (0.1517). At the maximum training set size of 100 samples, the PM method's mean RMSE of 0.0173 remains the lowest. This value is 23.79% lower than the next-best method, LHS (0.0227), and 51.13% lower than the worst-performing method, CVT (0.0354). Under these conditions, the maximum improvement was 67.27% over D-Optimality with 50 samples, and the minimum was 23.79% over LHS with 100 samples.

The PM method also demonstrates a substantial advantage in median RMSE. With a 40-sample training set, its median RMSE is 0.0310. This is 51.86% lower than the best competitor, LHS (0.0644), and 77.62% lower than the worst, D-Optimality (0.1385), indicating rapid and stable performance gains in typical scenarios. At 100 samples, the PM method's median RMSE of 0.0164 remains the lowest. This is 28.70% lower than the best competitor, LHS (0.0230), and 53.41% lower than the worst, CVT (0.0352). For this batch size, the greatest improvement in median RMSE was 77.62% over D-Optimality with 40 samples, and the smallest was 28.70% over LHS with 100 samples.

An analysis of the global response surface (Fig. 5) reveals that the PM method has a slightly higher RMSE (both mean and median) across the entire domain compared to other methods. This discrepancy arises because the proposed algorithm (Algorithm 1) preferentially allocates new sample points to the subregion containing the optimum, which can marginally reduce global predictive accuracy. However, a combined assessment of Figs. 2 and 3 demonstrates that this reduction in global accuracy is substantially outweighed by the gain in predictive accuracy within the optimal subregion. Therefore, the proposed method entails a minor trade-off in global precision to achieve a marked enhancement in localizing and characterizing the optimum. This targeted improvement substantially increases the practical utility of the robust parameter optimization model and reinforces the stability and reliability of the final solution.

## 4.2 Five-Dimensional Function Case

In a two-dimensional space, a function depends on only two independent variables. This low dimensionality simplifies the interactions between variables, enabling predictive models to effectively capture the function's global trends and local features. However, such simplified interaction patterns limit the applicability of these models to complex systems, as real-world problems typically involve nonlinear relationships among multiple variables. To demonstrate the applicability and superiority of the proposed method in high-dimensional settings, this case study employs a high-dimensional numerical function. The test function is defined as:

For the domain where , the function is a composite expression containing periodic components and nonlinear growth terms. Specifically, the terms andintroduce periodic oscillations along the and axes with periods of 2 and 1, respectively. Furthermore, the additional term introduces a linear effect, creating a monotonic trend in the function's value with respect to .

Conversely, in the domain where , the function simplifies to a form lacking this additional linear term. Here, its behavior is governed by the trigonometric functions, the quadratic term, and the exponential decay term . The quadratic term imparts symmetric growth along the axis, while the exponential term ensures monotonic decay asincreases. This combination of terms results in dynamic characteristics markedly different from those in the other domain.

Following the methodology of the previous section, Root Mean Square Error (RMSE) was again employed to evaluate the accuracy of the response surface model. The experimental procedure began by generating an initial set of 80 samples using Latin Hypercube Sampling (LHS) to fit a Non-Stationary Gaussian Process (NSGP) model. Subsequently, the proposed attention-based method was used to augment the training set with 20 additional samples, for a final total of 100. The NSGP model was then updated using this complete dataset. To compare the modeling accuracy of each method under challenging conditions of high-dimensional data sparsity and complex function behavior, this entire process was repeated 30 times. For a comprehensive benchmark, the four comparison methods were also used to generate datasets of the same sizes. The resulting RMSE statistics for all methods are summarized in Table 5 and illustrated in Fig. 6.

Table 5 RMSE statistics for the five-dimensional function over 30 independent runs

| Total sample size | Mean |  |  |  |  | Median |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM | D_opt | G_opt | LHS | CVT | PM |
| 100 | 0.4075 | 0.4146 | 0.4372 | 0.4372 | 0.3711 | 0.4107 | 0.4025 | 0.4360 | 0.4447 | 0.3793 |

Figure 6 Box plots of test results for the five-dimensional function

As shown in Table 5, the proposed method (PM) demonstrates superior performance. The PM method achieved a mean RMSE of 0.3711, the lowest among all tested approaches. This represents an improvement of approximately 8.93% over the next-best method, D-opt (mean 0.4075), and improvements of 10.49% and 15.12% relative to G-opt (mean 0.4146) and both LHS and CVT (mean 0.4372), respectively.

A similar trend was observed for the median RMSE, where the PM method's value of 0.3793 was also the lowest. It outperformed the runner-up G-opt (median 0.4025) by 5.76% and showed reductions of 7.65%, 13.00%, and 14.71% compared to D-opt, LHS, and CVT, respectively. This consistent lead in both central tendency and average performance underscores the stability and superiority of the PM method. These results suggest that the additional points selected by the proposed algorithm enhance sampling efficiency by targeting potential subregions of the optimum. This, in turn, yields a more effective response surface model for non-stationary parameter optimization, particularly when data is limited.

Notably, this performance advantage, while significant, is less pronounced than that observed for the two-dimensional test function. This is attributable to the challenge of high-dimensional data sparsity; even with the additional points, the sampling of the entire design space remains sparse. It is anticipated that the superiority of the PM method would become even more evident if the number of design points were further increased.

## 4.3 Lithium battery SOC prediction case

The State of Charge (SOC) of a rechargeable battery quantifies its current energy level relative to its maximum capacity, typically expressed as a percentage. Accurate SOC estimation is complicated by numerous factors, including temperature, aging (manifesting as capacity fade and increased internal resistance), charge/discharge C-rates, and self-discharge. These variables affect the battery's voltage response and available capacity, imparting significant non-stationary characteristics to the SOC dynamics. This makes it a suitable application for the proposed methodology. As a critical parameter reflecting battery health and performance, precise SOC estimation is essential for protecting the cell, preventing over-discharge, extending battery lifespan, and enabling applications to implement rational energy-saving control strategies[32]. Consequently, enhancing the accuracy of SOC estimation for lithium-ion batteries is of considerable importance.

The performance of the proposed method is demonstrated using a real-world case study on lithium-ion battery SOC estimation. The training and test data were sourced from a public dataset provided by Dr. Prashant of the University of Malaya[33]. This dataset comprises data from Electrochemical Impedance Spectroscopy (EIS) tests conducted on three types of commercial 18650 lithium-ion batteries. EIS is an effective experimental technique for evaluating the performance and degradation processes of lithium-ion batteries[34], and the 18650 designation refers to a common cylindrical cell format.

This case study involves one output response, the SOC, and three input parameters: temperature (°C), current (A), and voltage (V). A response surface model was first established to accurately capture the non-stationary functional relationship between the inputs and the output. An initial set of 180 training points was selected using Latin Hypercube Sampling (LHS). This set was then augmented with 30 additional points identified by the proposed attention-based algorithm, resulting in a total of 210 data samples. The entire experimental procedure was replicated 20 times to ensure statistical robustness. The results for the identified potential subregion of the optimum are presented in Table 6 and Fig. 7. For comparative analysis, Table 6 also includes the results obtained from the four alternative methods discussed previously.

Table 6 Summary of RMSE metrics from 20 experimental replications

| Method | Mean | Median | Max | Min | Q1 | Q3 |
| --- | --- | --- | --- | --- | --- | --- |
| G_opt | 7.9793 | 6.8957 | 18.3144 | 5.5627 | 6.0980 | 8.3195 |
| D_opt | 7.8077 | 7.0209 | 15.5626 | 5.9420 | 6.7881 | 7.6917 |
| CVT | 10.7557 | 9.2375 | 26.6414 | 6.9491 | 7.2289 | 12.0859 |
| LHS | 9.2725 | 7.6885 | 16.9593 | 6.4247 | 7.0761 | 10.5335 |
| PM | 6.9880 | 6.7256 | 10.9925 | 4.8605 | 6.1098 | 7.3933 |

Figure 7. Box plot illustrating the distribution of RMSE values for SOC prediction

The results in Table 6 demonstrate that the proposed method (PM) achieved a mean RMSE of 6.9880, outperforming all other evaluated methods. Specifically, the PM method yielded a 35.03% reduction in mean error relative to the poorest-performing method, CVT (mean = 10.7557), and a 10.50% reduction compared to the next-best method, D_opt (mean = 7.8077), confirming its superior overall performance.

Regarding central tendency, the PM method's median RMSE (6.7256) was also the lowest, indicating consistent high performance across the majority of experimental replications. This represents a 27.19% improvement in median performance over the CVT method (median = 9.2375). The superiority of the PM method across both mean and median metrics underscores its comprehensive lead in central tendency.

The robustness of the PM method is highlighted by its ability to mitigate extreme errors. It recorded the lowest maximum error (10.9925), which constitutes a 58.74% reduction in worst-case error compared to the CVT method's maximum of 26.6414. This result demonstrates the method's strong capacity to maintain a high performance baseline. Furthermore, the PM method also achieved the lowest minimum error (4.8605), surpassing all competitors and representing a 30.05% improvement over the highest minimum error from the CVT method (6.9491). This suggests efficient exploration of the solution space and a greater propensity to converge to high-quality solutions.

The stability of the PM method is also exceptional. Its Interquartile Range (IQR) of 1.2835 indicates a highly concentrated distribution of results, representing a 73.57% tighter distribution than that of the least stable method, CVT (IQR = 4.8570).

These statistical findings are visually corroborated by Figure 7. The box plot for the PM method is positioned lower than those of the other methods, features a narrower interquartile range, and has fewer outliers, all of which signify lower prediction error and greater stability.

## 4.4 Aerospace rocket optimization case

This section presents a case study in launch vehicle design to illustrate the practical efficacy and performance of the proposed method for robust parameter optimization. The core objective is to enhance vehicle performance by optimizing key design parameters within a complex system. All simulation data were generated using OpenRocket, an open-source simulator that models flight trajectories based on user-defined design parameters. The rocket model consists of a nose cone, a body tube, a recovery system (comprising a shock cord and parachute), an inner tube, and stabilizing fins. A detailed schematic of the model is presented in Figure 8.

Figure 8  Schematic of the launch vehicle model

- Launch vehicle flight trajectory                              (b) Effect of body tube diameter on apogee

Figure 9 Launch vehicle simulation results

The case study involves a single output response: apogee, or maximum flight hight, denoted as y(m). As illustrated by the flight trajectory in Fig. 9a, apogee is a critical performance metric that reflects ascent velocity and descent stability. It serves as a key indicator for evaluating the efficiency of the launch vehicle design and propulsion system. Analysis of the apogee provides insights into thrust, aerodynamic drag, and structural optimization, while also influencing the design and deployment of the recovery system and defining the vehicle's mission capabilities. For this study, a target apogee  of 400 m was selected to ensure a safe, controllable, and readily achievable test flight.

The model includes five controllable input parameters: nose cone length (0–20 cm), body tube outer diameter(2–5 cm), nose cone wall thickness (0–1 cm), body tube length (30–50 cm), and body tube inner diameter (1–2 cm). An additional categorical parameter, , defines the nose cone geometry, where .corresponds to a tangent ogive shape and corresponds to a secant ogive shape. The selection of nose cone geometry is conditional on its length () to optimize flight stability.For cm, a tangent ogive is used to reduce wave drag during supersonic flight by improving airflow and minimizing turbulence. For cm, a secant ogive is employed to enhance stability during subsonic flight by increasing lift. This conditional relationship introduces a distinct non-stationarity into the system, creating a step change in the output response atcm. The objective of the experiment is to identify the optimal combination of input parameters that enables the launch vehicle to achieve the target apogee of 400 m.

o accurately capture the non-stationary input-output relationship, a response surface model incorporating an attention mechanism was developed. An initial training set of 30 points was generated using Latin Hypercube Sampling (LHS). The proposed mixed-attention mechanism algorithm was then used to sequentially add 20 informative sample points, resulting in a total of 50 experimental data samples. The results are presented in Tables 7–9, which also include outcomes from four alternative methods for comparative analysis.

Table 7 The 50-sample dataset generated using Latin Hypercube Sampling (LHS) and the proposed method

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM |
| 1 | 15.5 | 15.5 | 3.13 | 3.13 | 0.96 | 0.96 | 48.8 | 48.8 | 1.89 | 1.89 | 1 | 1 | 26.7 | 26.7 |
| 2 | 12.9 | 12.9 | 2.81 | 2.81 | 0.07 | 0.07 | 41.3 | 41.3 | 1.96 | 1.96 | 1 | 1 | 130 | 130 |
| 3 | 1.3 | 1.3 | 2.06 | 2.06 | 0.63 | 0.63 | 37.4 | 37.4 | 1.31 | 1.31 | 0 | 0 | 227 | 227 |
| 4 | 0.5 | 0.5 | 3.97 | 3.97 | 0.1 | 0.1 | 45.6 | 45.6 | 1.23 | 1.23 | 0 | 0 | 7.59 | 7.59 |
| 5 | 10.7 | 10.7 | 4.17 | 4.17 | 0.13 | 0.13 | 33.9 | 33.9 | 1.86 | 1.86 | 1 | 1 | 1.1 | 1.1 |
| 6 | 18.7 | 18.7 | 4.81 | 4.81 | 0.86 | 0.86 | 36.7 | 36.7 | 1.18 | 1.18 | 1 | 1 | 0.39 | 0.39 |
| 7 | 17.9 | 17.9 | 3.85 | 3.85 | 0.65 | 0.65 | 47 | 47 | 1.49 | 1.49 | 1 | 1 | 4.59 | 4.59 |
| 8 | 2.6 | 2.6 | 2.39 | 2.39 | 0.83 | 0.83 | 45.5 | 45.5 | 1.53 | 1.53 | 0 | 0 | 146 | 146 |
| 9 | 2 | 2 | 4.69 | 4.69 | 0.67 | 0.67 | 32.9 | 32.9 | 1.15 | 1.15 | 0 | 0 | 5.65 | 5.65 |
| 10 | 11.1 | 11.1 | 4.97 | 4.97 | 0.14 | 0.14 | 44.2 | 44.2 | 1.92 | 1.92 | 1 | 1 | 0.64 | 0.64 |
| 11 | 18 | 18 | 3.99 | 3.99 | 0.82 | 0.82 | 40.6 | 40.6 | 1.98 | 1.98 | 1 | 1 | 7.19 | 7.19 |
| 12 | 2 | 2 | 2.96 | 2.96 | 0.24 | 0.24 | 42.6 | 42.6 | 1.33 | 1.33 | 0 | 0 | 6.25 | 6.25 |
| 13 | 17.3 | 17.3 | 4.63 | 4.63 | 0.91 | 0.91 | 43.3 | 43.3 | 1 | 1 | 1 | 1 | 0.27 | 0.27 |
| 14 | 11.2 | 11.2 | 3.71 | 3.71 | 0.32 | 0.32 | 33.5 | 33.5 | 1.72 | 1.72 | 1 | 1 | 29 | 29 |
| 15 | 8.1 | 8.1 | 3.75 | 3.75 | 0.5 | 0.5 | 35.8 | 35.8 | 1.69 | 1.69 | 0 | 0 | 24.6 | 24.6 |
| 16 | 15.6 | 15.6 | 2.5 | 2.5 | 0.23 | 0.23 | 47.4 | 47.4 | 1.26 | 1.26 | 1 | 1 | 77.6 | 77.6 |
| 17 | 7.5 | 7.5 | 3.15 | 3.15 | 0.44 | 0.44 | 31.5 | 31.5 | 1.57 | 1.57 | 0 | 0 | 73.3 | 73.3 |
| 18 | 8.8 | 8.8 | 3.45 | 3.45 | 0.7 | 0.7 | 44.6 | 44.6 | 1.81 | 1.81 | 0 | 0 | 25.9 | 25.9 |
| 19 | 19 | 19 | 2.26 | 2.26 | 0.04 | 0.04 | 49.5 | 49.5 | 1.47 | 1.47 | 1 | 1 | 159 | 159 |
| 20 | 6.8 | 6.8 | 4.13 | 4.13 | 0.5 | 0.5 | 36.1 | 36.1 | 1.44 | 1.44 | 0 | 0 | 11.8 | 11.8 |
| 21 | 11.9 | 11.9 | 3.55 | 3.55 | 0.84 | 0.84 | 42 | 42 | 1.74 | 1.74 | 1 | 1 | 17.3 | 17.3 |
| 22 | 6.6 | 6.6 | 3.58 | 3.58 | 0.8 | 0.8 | 37.9 | 37.9 | 1.06 | 1.06 | 0 | 0 | 19.4 | 19.4 |
| 23 | 2.9 | 2.9 | 2.01 | 2.01 | 0.76 | 0.76 | 47.8 | 47.8 | 1.74 | 1.74 | 0 | 0 | 306 | 306 |
| 24 | 13.4 | 13.4 | 2.35 | 2.35 | 0.1 | 0.1 | 39.8 | 39.8 | 1.22 | 1.22 | 1 | 1 | 139 | 139 |
| 25 | 8.7 | 8.7 | 2.18 | 2.18 | 0.54 | 0.54 | 40.2 | 40.2 | 1.03 | 1.03 | 0 | 0 | 147 | 147 |
| 26 | 9.9 | 9.9 | 2.2 | 2.2 | 0.02 | 0.02 | 38.4 | 38.4 | 1.62 | 1.62 | 0 | 0 | 262 | 262 |
| 27 | 14.2 | 14.2 | 2.91 | 2.91 | 0.28 | 0.28 | 38.6 | 38.6 | 1.86 | 1.86 | 1 | 1 | 87.5 | 87.5 |
| 28 | 16.6 | 16.6 | 2.71 | 2.71 | 0.9 | 0.9 | 40.9 | 40.9 | 1.38 | 1.38 | 1 | 1 | 50.5 | 50.5 |
| 29 | 17.2 | 17.2 | 3.32 | 3.32 | 0.44 | 0.44 | 30.8 | 30.8 | 1.14 | 1.14 | 1 | 1 | 33.6 | 33.6 |
| 30 | 5.2 | 5.2 | 4.5 | 4.5 | 1 | 1 | 32 | 32 | 1.54 | 1.54 | 0 | 0 | 9.25 | 9.25 |
| 31 | 12.5 | 0.7 | 4.35 | 2.08 | 0.3 | 0.25 | 44.9 | 30.7 | 1.5 | 1.92 | 1 | 0 | 2.6 | 373.89 |
| 32 | 4.8 | 4.9 | 3.65 | 2.03 | 0.62 | 0.16 | 35.2 | 34.8 | 1.35 | 1.88 | 0 | 0 | 25.5 | 358.91 |
| 33 | 3.9 | 5.8 | 3.07 | 2.12 | 0.38 | 0.02 | 42.3 | 30.2 | 1.97 | 1.97 | 0 | 0 | 79.2 | 369.85 |
| 34 | 1.1 | 8.7 | 3.26 | 2.02 | 0.46 | 0.19 | 35.5 | 33.4 | 1.83 | 1.89 | 0 | 0 | 66 | 362.51 |
| 35 | 6.3 | 10.5 | 4.74 | 2.03 | 0.19 | 0.35 | 39 | 31.3 | 1.77 | 1.86 | 0 | 1 | 1.03 | 358.85 |
| 36 | 13.9 | 0.4 | 4.05 | 2.04 | 0.06 | 0.77 | 30.7 | 34.9 | 1.9 | 1.95 | 1 | 0 | 1.12 | 365.41 |
| 37 | 16.3 | 4.0 | 4.32 | 2.05 | 0.37 | 0.17 | 34.3 | 32.8 | 1.11 | 1.85 | 1 | 0 | 1.07 | 358.41 |
| 38 | 3.5 | 2.1 | 4.9 | 2.08 | 0.21 | 0.45 | 37.2 | 35.6 | 1.65 | 1.99 | 0 | 0 | 2.72 | 359.29 |
| 39 | 1.7 | 6.8 | 4.85 | 2.12 | 0.34 | 0.07 | 42.9 | 33.5 | 1.43 | 1.99 | 0 | 0 | 0.793 | 358.02 |
| 40 | 19.4 | 10.3 | 2.74 | 2.03 | 0.92 | 0.08 | 46.3 | 36.1 | 1.1 | 1.89 | 1 | 1 | 29.5 | 348.89 |
| 41 | 9.9 | 2.0 | 2.61 | 2.04 | 0.52 | 0.89 | 32.8 | 33.6 | 1.18 | 1.84 | 0 | 0 | 109 | 349.30 |
| 42 | 9.3 | 3.0 | 4.54 | 2.13 | 0.28 | 0.33 | 30 | 31.3 | 1.79 | 1.87 | 0 | 0 | 1.05 | 347.57 |
| 43 | 7.7 | 8.1 | 3.4 | 2.04 | 0.77 | 0.85 | 49.7 | 35.0 | 1.67 | 1.90 | 0 | 0 | 20.1 | 345.66 |
| 44 | 19.7 | 4.3 | 2.59 | 2.12 | 0.97 | 1.00 | 39.5 | 30.4 | 1.42 | 1.90 | 1 | 0 | 59.8 | 350.13 |
| 45 | 4.8 | 7.8 | 2.47 | 2.00 | 0.16 | 0.93 | 43.9 | 30.4 | 1.6 | 1.82 | 0 | 0 | 149 | 360.47 |
| 46 | 4.1 | 8.4 | 3.36 | 2.02 | 0.6 | 0.55 | 49 | 32.3 | 1.4 | 1.89 | 0 | 0 | 20 | 365.20 |
| 47 | 5.9 | 11.8 | 3.87 | 2.09 | 0.56 | 0.14 | 32.4 | 32.0 | 1.29 | 1.86 | 0 | 1 | 21.5 | 340.60 |
| 48 | 2.2 | 19.8 | 4.45 | 2.01 | 0.71 | 0.82 | 48.3 | 34.2 | 1.26 | 1.97 | 0 | 1 | 1.07 | 353.39 |
| 49 | 14.7 | 12.9 | 4.26 | 2.04 | 0.73 | 0.72 | 46.5 | 35.0 | 1.06 | 1.90 | 1 | 1 | 0.777 | 342.23 |
| 50 | 12.3 | 14.0 | 2.87 | 2.00 | 0.41 | 0.40 | 34.5 | 36.3 | 1.63 | 1.88 | 1 | 1 | 84.3 | 341.54 |

Table 8 The 50-sample dataset generated using G_opt and D_opt

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt |
| 1 | 15.5 | 15.5 | 3.13 | 3.13 | 0.96 | 0.96 | 48.8 | 48.8 | 1.89 | 1.89 | 1 | 1 | 26.7 | 26.7 |
| 2 | 12.9 | 12.9 | 2.81 | 2.81 | 0.07 | 0.07 | 41.3 | 41.3 | 1.96 | 1.96 | 1 | 1 | 130.0 | 130.0 |
| 3 | 1.3 | 1.3 | 2.06 | 2.06 | 0.63 | 0.63 | 37.4 | 37.4 | 1.31 | 1.31 | 0 | 0 | 227.0 | 227.0 |
| 4 | 0.5 | 0.5 | 3.97 | 3.97 | 0.10 | 0.10 | 45.6 | 45.6 | 1.23 | 1.23 | 0 | 0 | 7.6 | 7.6 |
| 5 | 10.7 | 10.7 | 4.17 | 4.17 | 0.13 | 0.13 | 33.9 | 33.9 | 1.86 | 1.86 | 1 | 1 | 1.1 | 1.1 |
| 6 | 18.7 | 18.7 | 4.81 | 4.81 | 0.86 | 0.86 | 36.7 | 36.7 | 1.18 | 1.18 | 1 | 1 | 0.4 | 0.4 |
| 7 | 17.9 | 17.9 | 3.85 | 3.85 | 0.65 | 0.65 | 47.0 | 47.0 | 1.49 | 1.49 | 1 | 1 | 4.6 | 4.6 |
| 8 | 2.6 | 2.6 | 2.39 | 2.39 | 0.83 | 0.83 | 45.5 | 45.5 | 1.53 | 1.53 | 0 | 0 | 146.0 | 146.0 |
| 9 | 2.0 | 2.0 | 4.69 | 4.69 | 0.67 | 0.67 | 32.9 | 32.9 | 1.15 | 1.15 | 0 | 0 | 5.7 | 5.7 |
| 10 | 11.1 | 11.1 | 4.97 | 4.97 | 0.14 | 0.14 | 44.2 | 44.2 | 1.92 | 1.92 | 1 | 1 | 0.6 | 0.6 |
| 11 | 18.0 | 18.0 | 3.99 | 3.99 | 0.82 | 0.82 | 40.6 | 40.6 | 1.98 | 1.98 | 1 | 1 | 7.2 | 7.2 |
| 12 | 2.0 | 2.0 | 2.96 | 2.96 | 0.24 | 0.24 | 42.6 | 42.6 | 1.33 | 1.33 | 0 | 0 | 6.3 | 6.3 |
| 13 | 17.3 | 17.3 | 4.63 | 4.63 | 0.91 | 0.91 | 43.3 | 43.3 | 1.00 | 1.00 | 1 | 1 | 0.3 | 0.3 |
| 14 | 11.2 | 11.2 | 3.71 | 3.71 | 0.32 | 0.32 | 33.5 | 33.5 | 1.72 | 1.72 | 1 | 1 | 29.0 | 29.0 |
| 15 | 8.1 | 8.1 | 3.75 | 3.75 | 0.50 | 0.50 | 35.8 | 35.8 | 1.69 | 1.69 | 0 | 0 | 24.6 | 24.6 |
| 16 | 15.6 | 15.6 | 2.50 | 2.50 | 0.23 | 0.23 | 47.4 | 47.4 | 1.26 | 1.26 | 1 | 1 | 77.6 | 77.6 |
| 17 | 7.5 | 7.5 | 3.15 | 3.15 | 0.44 | 0.44 | 31.5 | 31.5 | 1.57 | 1.57 | 0 | 0 | 73.3 | 73.3 |
| 18 | 8.8 | 8.8 | 3.45 | 3.45 | 0.70 | 0.70 | 44.6 | 44.6 | 1.81 | 1.81 | 0 | 0 | 25.9 | 25.9 |
| 19 | 19.0 | 19.0 | 2.26 | 2.26 | 0.04 | 0.04 | 49.5 | 49.5 | 1.47 | 1.47 | 1 | 1 | 159.0 | 159.0 |
| 20 | 6.8 | 6.8 | 4.13 | 4.13 | 0.50 | 0.50 | 36.1 | 36.1 | 1.44 | 1.44 | 0 | 0 | 11.8 | 11.8 |
| 21 | 11.9 | 11.9 | 3.55 | 3.55 | 0.84 | 0.84 | 42.0 | 42.0 | 1.74 | 1.74 | 1 | 1 | 17.3 | 17.3 |
| 22 | 6.6 | 6.6 | 3.58 | 3.58 | 0.80 | 0.80 | 37.9 | 37.9 | 1.06 | 1.06 | 0 | 0 | 19.4 | 19.4 |
| 23 | 2.9 | 2.9 | 2.01 | 2.01 | 0.76 | 0.76 | 47.8 | 47.8 | 1.74 | 1.74 | 0 | 0 | 306.0 | 306.0 |
| 24 | 13.4 | 13.4 | 2.35 | 2.35 | 0.10 | 0.10 | 39.8 | 39.8 | 1.22 | 1.22 | 1 | 1 | 139.0 | 139.0 |
| 25 | 8.7 | 8.7 | 2.18 | 2.18 | 0.54 | 0.54 | 40.2 | 40.2 | 1.03 | 1.03 | 0 | 0 | 147.0 | 147.0 |
| 26 | 9.9 | 9.9 | 2.20 | 2.20 | 0.02 | 0.02 | 38.4 | 38.4 | 1.62 | 1.62 | 0 | 0 | 262.0 | 262.0 |
| 27 | 14.2 | 14.2 | 2.91 | 2.91 | 0.28 | 0.28 | 38.6 | 38.6 | 1.86 | 1.86 | 1 | 1 | 87.5 | 87.5 |
| 28 | 16.6 | 16.6 | 2.71 | 2.71 | 0.90 | 0.90 | 40.9 | 40.9 | 1.38 | 1.38 | 1 | 1 | 50.5 | 50.5 |
| 29 | 17.2 | 17.2 | 3.32 | 3.32 | 0.44 | 0.44 | 30.8 | 30.8 | 1.14 | 1.14 | 1 | 1 | 33.6 | 33.6 |
| 30 | 5.2 | 5.2 | 4.50 | 4.50 | 1.00 | 1.00 | 32.0 | 32.0 | 1.54 | 1.54 | 0 | 0 | 9.3 | 9.3 |
| 31 | 1.0 | 20.0 | 4.58 | 3.24 | 0.11 | 0.16 | 49.2 | 49.4 | 1.11 | 1.02 | 0 | 1 | -10.0 | -89.4 |
| 32 | 0.3 | 5.1 | 2.23 | 3.30 | 0.73 | 0.88 | 30.3 | 31.3 | 1.99 | 1.08 | 0 | 0 | 312.0 | 39.2 |
| 33 | 17.7 | 18.3 | 2.09 | 2.06 | 0.05 | 0.87 | 31.4 | 35.6 | 1.02 | 1.89 | 1 | 1 | 186.3 | 293.1 |
| 34 | 19.9 | 0.3 | 4.95 | 4.46 | 0.08 | 0.91 | 31.7 | 47.6 | 1.91 | 1.91 | 1 | 0 | -4.1 | 11.1 |
| 35 | 19.3 | 18.9 | 4.99 | 4.77 | 1.00 | 0.03 | 49.2 | 48.2 | 1.03 | 1.07 | 1 | 1 | -31.5 | -24.0 |
| 36 | 0.8 | 0.5 | 4.67 | 3.19 | 0.10 | 0.21 | 48.8 | 31.3 | 1.83 | 1.92 | 0 | 0 | 4.1 | 112.4 |
| 37 | 3.8 | 17.9 | 4.95 | 4.98 | 0.00 | 0.98 | 31.0 | 49.5 | 1.03 | 1.76 | 0 | 1 | -13.2 | -17.2 |
| 38 | 0.4 | 20.0 | 3.05 | 3.19 | 0.98 | 0.33 | 30.4 | 30.1 | 1.99 | 1.05 | 0 | 1 | 143.7 | -18.6 |
| 39 | 19.4 | 6.3 | 4.49 | 3.33 | 0.99 | 0.19 | 30.1 | 38.9 | 1.97 | 1.07 | 1 | 0 | 6.9 | 17.2 |
| 40 | 1.1 | 4.9 | 4.75 | 3.20 | 0.90 | 0.09 | 49.7 | 41.8 | 1.10 | 1.03 | 0 | 0 | -15.4 | -54.4 |
| 41 | 11.2 | 8.4 | 2.14 | 3.37 | 0.99 | 0.41 | 30.1 | 47.6 | 1.03 | 1.08 | 1 | 0 | 175.5 | 4.0 |
| 42 | 20.0 | 6.5 | 2.08 | 2.63 | 0.81 | 0.99 | 49.8 | 30.6 | 1.91 | 1.95 | 1 | 0 | 250.7 | 204.1 |
| 43 | 19.1 | 4.8 | 4.79 | 3.34 | 0.05 | 0.34 | 30.3 | 46.0 | 1.11 | 1.06 | 1 | 0 | -11.6 | 7.1 |
| 44 | 1.2 | 18.2 | 2.18 | 3.32 | 0.00 | 0.22 | 49.9 | 47.2 | 1.86 | 1.12 | 0 | 1 | 268.6 | 7.5 |
| 45 | 1.0 | 3.6 | 4.73 | 3.20 | 0.14 | 0.88 | 30.0 | 37.5 | 1.97 | 1.02 | 0 | 0 | 13.2 | -36.7 |
| 46 | 18.8 | 2.2 | 4.76 | 3.13 | 0.01 | 0.47 | 48.3 | 44.4 | 1.87 | 1.00 | 1 | 0 | -5.2 | -56.7 |
| 47 | 16.9 | 5.6 | 2.03 | 3.31 | 0.00 | 0.59 | 34.2 | 48.3 | 1.02 | 1.07 | 1 | 0 | 192.0 | 4.6 |
| 48 | 1.8 | 13.9 | 4.98 | 3.20 | 0.98 | 0.62 | 48.5 | 45.6 | 1.19 | 1.04 | 0 | 1 | -21.9 | -75.5 |
| 49 | 1.3 | 18.9 | 2.21 | 4.48 | 0.05 | 0.07 | 48.7 | 30.0 | 1.05 | 1.24 | 0 | 1 | 136.3 | -2.2 |
| 50 | 1.0 | 9.9 | 2.15 | 3.51 | 0.88 | 0.71 | 30.2 | 43.7 | 1.86 | 1.02 | 0 | 0 | 307.3 | 7.6 |

Table 9 The 50-sample dataset generated using CVT

| NO | X1 | X2 | X3 | X4 | X5 | X6 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CVT | CVT | CVT | CVT | CVT | CVT | CVT |
| 1 | 15.5 | 3.13 | 0.96 | 48.8 | 1.89 | 1 | 26.7 |
| 2 | 12.9 | 2.81 | 0.07 | 41.3 | 1.96 | 1 | 130.0 |
| 3 | 1.3 | 2.06 | 0.63 | 37.4 | 1.31 | 0 | 227.0 |
| 4 | 0.5 | 3.97 | 0.10 | 45.6 | 1.23 | 0 | 7.6 |
| 5 | 10.7 | 4.17 | 0.13 | 33.9 | 1.86 | 1 | 1.1 |
| 6 | 18.7 | 4.81 | 0.86 | 36.7 | 1.18 | 1 | 0.4 |
| 7 | 17.9 | 3.85 | 0.65 | 47.0 | 1.49 | 1 | 4.6 |
| 8 | 2.6 | 2.39 | 0.83 | 45.5 | 1.53 | 0 | 146.0 |
| 9 | 2.0 | 4.69 | 0.67 | 32.9 | 1.15 | 0 | 5.7 |
| 10 | 11.1 | 4.97 | 0.14 | 44.2 | 1.92 | 1 | 0.6 |
| 11 | 18.0 | 3.99 | 0.82 | 40.6 | 1.98 | 1 | 7.2 |
| 12 | 2.0 | 2.96 | 0.24 | 42.6 | 1.33 | 0 | 6.3 |
| 13 | 17.3 | 4.63 | 0.91 | 43.3 | 1.00 | 1 | 0.3 |
| 14 | 11.2 | 3.71 | 0.32 | 33.5 | 1.72 | 1 | 29.0 |
| 15 | 8.1 | 3.75 | 0.50 | 35.8 | 1.69 | 0 | 24.6 |
| 16 | 15.6 | 2.50 | 0.23 | 47.4 | 1.26 | 1 | 77.6 |
| 17 | 7.5 | 3.15 | 0.44 | 31.5 | 1.57 | 0 | 73.3 |
| 18 | 8.8 | 3.45 | 0.70 | 44.6 | 1.81 | 0 | 25.9 |
| 19 | 19.0 | 2.26 | 0.04 | 49.5 | 1.47 | 1 | 159.0 |
| 20 | 6.8 | 4.13 | 0.50 | 36.1 | 1.44 | 0 | 11.8 |
| 21 | 11.9 | 3.55 | 0.84 | 42.0 | 1.74 | 1 | 17.3 |
| 22 | 6.6 | 3.58 | 0.80 | 37.9 | 1.06 | 0 | 19.4 |
| 23 | 2.9 | 2.01 | 0.76 | 47.8 | 1.74 | 0 | 306.0 |
| 24 | 13.4 | 2.35 | 0.10 | 39.8 | 1.22 | 1 | 139.0 |
| 25 | 8.7 | 2.18 | 0.54 | 40.2 | 1.03 | 0 | 147.0 |
| 26 | 9.9 | 2.20 | 0.02 | 38.4 | 1.62 | 0 | 262.0 |
| 27 | 14.2 | 2.91 | 0.28 | 38.6 | 1.86 | 1 | 87.5 |
| 28 | 16.6 | 2.71 | 0.90 | 40.9 | 1.38 | 1 | 50.5 |
| 29 | 17.2 | 3.32 | 0.44 | 30.8 | 1.14 | 1 | 33.6 |
| 30 | 5.2 | 4.50 | 1.00 | 32.0 | 1.54 | 0 | 9.3 |
| 31 | 17.9 | 3.47 | 0.50 | 37.8 | 1.51 | 1 | 20.5 |
| 32 | 5.9 | 3.55 | 0.51 | 40.5 | 1.50 | 0 | 17.4 |
| 33 | 6.3 | 3.49 | 0.51 | 31.8 | 1.49 | 0 | 34.1 |
| 34 | 17.6 | 3.51 | 0.48 | 47.8 | 1.50 | 1 | 5.3 |
| 35 | 14.1 | 3.47 | 0.52 | 31.7 | 1.50 | 1 | 33.7 |
| 36 | 9.9 | 3.51 | 0.50 | 38.4 | 1.50 | 0 | 26.9 |
| 37 | 2.5 | 3.46 | 0.51 | 42.9 | 1.48 | 0 | 18.5 |
| 38 | 9.1 | 3.50 | 0.49 | 43.7 | 1.51 | 0 | 13.8 |
| 39 | 1.9 | 3.61 | 0.49 | 37.8 | 1.50 | 0 | 33.7 |
| 40 | 13.8 | 3.61 | 0.50 | 44.8 | 1.49 | 1 | 13.9 |
| 41 | 13.8 | 3.46 | 0.51 | 40.6 | 1.50 | 1 | -6.6 |
| 42 | 11.7 | 3.52 | 0.51 | 48.1 | 1.48 | 1 | 9.8 |
| 43 | 10.1 | 3.48 | 0.51 | 32.8 | 1.52 | 1 | 36.3 |
| 44 | 13.8 | 3.48 | 0.50 | 35.9 | 1.51 | 1 | 29.3 |
| 45 | 18.0 | 3.48 | 0.50 | 32.7 | 1.49 | 1 | 26.9 |
| 46 | 6.0 | 3.50 | 0.48 | 35.8 | 1.50 | 0 | 35.8 |
| 47 | 2.0 | 3.51 | 0.51 | 32.5 | 1.50 | 0 | 55.2 |
| 48 | 6.5 | 3.49 | 0.49 | 47.5 | 1.50 | 0 | 11.8 |
| 49 | 18.1 | 3.48 | 0.51 | 42.8 | 1.51 | 1 | 21.3 |
| 50 | 2.1 | 3.42 | 0.49 | 47.7 | 1.51 | 0 | 14.4 |

As shown in Tables 7–9, the 20 sequentially added samples (points 31–50) generated by the proposed method consistently yield outputs near the target value of 400. This result, which contrasts with those from the four alternative methods, indicates that the proposed approach can accurately characterize the function's behavior within the potential optimal subregion. Furthermore, an analysis of the input space reveals that these additional sample points are well-distributed, without significant clustering or overlap, which further validates the method's effectiveness. To provide a more comprehensive demonstration of these advantages, additional simulations were conducted using initial training sets of 40, 50, and 60 points, each augmented by 20 sequential samples, resulting in total datasets of 60, 70, and 80 points, respectively. The detailed results are presented in Appendix C.

After the final experimental dataset is obtained, the Non-Stationary Gaussian Process (NSGP) model is updated. The output from this refined model is then used to formulate the robust parameter optimization model as follows:

Where,represents the predicted apogee from the Non-Stationary Gaussian Process (NSGP) model, is the predictive standard deviation, and the target value is set to 400. A genetic algorithm (GA) is then employed to minimize this objective function (Eq.33)and identify the optimal input parameter configuration. The GA was configured with the following hyperparameters: a population size of 50, a maximum of 200 generations, and a stall generation limit of 20. All other hyperparameters were set to their default values. To ensure the robustness and validity of the findings, the optimization process was repeated 30 times for each method. The results from these 30 independent runs, along with the corresponding simulated flight trajectories, are presented in Figs. 10–11 and Tables 10–13.

Table 10  Comparison of optimization performance between the proposed method (PM) and LHS over 30 independent runs

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM |
| 1 | 3.9 | 5.4 | 2.20 | 2.02 | 0.02 | 0.27 | 41.5 | 33.3 | 1.80 | 1.92 | 0 | 0 | 287 | 381 |
| 2 | 8.2 | 6.4 | 2.09 | 2.04 | 0.27 | 0.41 | 39.8 | 30.2 | 1.95 | 1.98 | 0 | 0 | 351 | 389 |
| 3 | 6.0 | 5.5 | 2.12 | 2.08 | 0.12 | 0.26 | 33.7 | 30.1 | 1.90 | 1.92 | 0 | 0 | 356 | 373 |
| 4 | 8.5 | 5.7 | 2.06 | 2.05 | 0.09 | 0.22 | 33.8 | 34.1 | 1.82 | 1.96 | 0 | 0 | 356 | 379 |
| 5 | 2.3 | 6.9 | 2.00 | 2.05 | 0.36 | 0.40 | 30.4 | 31.3 | 1.78 | 1.98 | 0 | 0 | 356 | 382 |
| 6 | 2.8 | 12.2 | 2.10 | 2.06 | 0.52 | 0.44 | 31.2 | 30.7 | 1.86 | 1.97 | 0 | 1 | 346 | 346 |
| 7 | 7.7 | 5.1 | 2.09 | 2.04 | 0.18 | 0.32 | 37.2 | 30.2 | 1.91 | 1.95 | 0 | 0 | 354 | 384 |
| 8 | 7.4 | 5.4 | 2.11 | 2.02 | 0.19 | 0.28 | 34.5 | 39.3 | 1.96 | 1.95 | 0 | 0 | 361 | 373 |
| 9 | 4.1 | 14.8 | 2.07 | 2.06 | 0.48 | 0.46 | 33.1 | 32.6 | 1.92 | 1.98 | 0 | 1 | 365 | 330 |
| 10 | 7.3 | 2.5 | 2.06 | 2.08 | 0.17 | 0.14 | 36.3 | 31.7 | 1.88 | 1.90 | 0 | 0 | 357 | 355 |
| 11 | 1.4 | 1.8 | 2.04 | 2.03 | 0.25 | 0.38 | 32.7 | 32.0 | 1.86 | 1.95 | 0 | 0 | 331 | 357 |
| 12 | 2.2 | 2.2 | 2.10 | 2.03 | 0.18 | 0.14 | 39.3 | 37.7 | 1.89 | 1.98 | 0 | 0 | 325 | 360 |
| 13 | 7.3 | 1.6 | 2.18 | 2.06 | 0.22 | 0.13 | 31.7 | 33.3 | 1.94 | 1.99 | 0 | 0 | 345 | 346 |
| 14 | 6.5 | 4.8 | 2.07 | 2.01 | 0.08 | 0.10 | 36.4 | 37.1 | 1.93 | 1.98 | 0 | 0 | 369 | 385 |
| 15 | 6.2 | 11.5 | 2.02 | 2.05 | 0.22 | 0.10 | 38.7 | 30.9 | 1.84 | 1.92 | 0 | 1 | 354 | 373 |
| 16 | 1.8 | 1.9 | 2.08 | 2.10 | 0.21 | 0.29 | 38.7 | 33.6 | 1.91 | 1.97 | 0 | 0 | 328 | 343 |
| 17 | 4.2 | 5.6 | 2.06 | 2.07 | 0.42 | 0.33 | 31.9 | 31.6 | 1.87 | 1.88 | 0 | 0 | 361 | 362 |
| 18 | 3.4 | 4.6 | 2.05 | 2.02 | 0.27 | 0.18 | 30.8 | 36.3 | 1.88 | 1.96 | 0 | 0 | 369 | 382 |
| 19 | 4.4 | 4.8 | 2.04 | 2.06 | 0.37 | 0.22 | 31.1 | 34.5 | 1.93 | 1.94 | 0 | 0 | 379 | 372 |
| 20 | 8.7 | 7.3 | 2.06 | 2.03 | 0.35 | 0.02 | 33.5 | 36.5 | 1.95 | 1.91 | 0 | 0 | 370 | 378 |
| 21 | 5.7 | 1.3 | 2.19 | 2.02 | 0.45 | 0.27 | 31.6 | 32.1 | 1.91 | 1.91 | 0 | 0 | 333 | 334 |
| 22 | 4.3 | 1.2 | 2.01 | 2.16 | 0.17 | 0.15 | 36.2 | 32.6 | 1.91 | 1.96 | 0 | 0 | 377 | 303 |
| 23 | 7.5 | 1.9 | 2.09 | 2.07 | 0.25 | 0.27 | 33.9 | 32.3 | 1.97 | 1.93 | 0 | 0 | 367 | 346 |
| 24 | 9.5 | 7.2 | 2.09 | 2.03 | 0.20 | 0.31 | 34.8 | 30.9 | 1.92 | 1.99 | 0 | 0 | 356 | 390 |
| 25 | 3.1 | 7.1 | 2.05 | 2.05 | 0.52 | 0.22 | 31.8 | 33.8 | 1.86 | 1.99 | 0 | 0 | 360 | 385 |
| 26 | 1.8 | 7.7 | 2.25 | 2.16 | 0.58 | 0.62 | 32.0 | 30.9 | 1.90 | 1.95 | 0 | 0 | 292 | 341 |
| 27 | 5.8 | 3.5 | 2.07 | 2.08 | 0.26 | 0.13 | 37.7 | 30.3 | 1.94 | 1.98 | 0 | 0 | 361 | 380 |
| 28 | 7.6 | 4.2 | 2.07 | 2.07 | 0.14 | 0.04 | 38.5 | 32.8 | 1.93 | 1.94 | 0 | 0 | 362 | 376 |
| 29 | 5.8 | 2.1 | 2.10 | 2.01 | 0.21 | 0.37 | 34.3 | 30.4 | 1.98 | 1.96 | 0 | 0 | 368 | 372 |
| 30 | 7.5 | 4.3 | 2.15 | 2.03 | 0.19 | 0.12 | 35.3 | 31.8 | 1.98 | 1.99 | 0 | 0 | 352 | 391 |

Table 11 Comparison of optimization performance between D_opt and G_opt over 30 independent runs

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt |
| 1 | 5.7 | 6.0 | 2.03 | 2.12 | 0.19 | 0.18 | 41.0 | 38.9 | 1.83 | 1.89 | 0 | 0 | 345 | 333 |
| 2 | 2.2 | 10.3 | 2.10 | 2.25 | 0.82 | 0.40 | 30.3 | 30.6 | 1.77 | 1.84 | 0 | 1 | 324 | 275 |
| 3 | 10.2 | 4.3 | 2.05 | 2.02 | 0.30 | 0.81 | 31.4 | 31.4 | 1.85 | 1.91 | 1 | 0 | 344 | 379 |
| 4 | 7.3 | 2.5 | 2.06 | 2.11 | 0.07 | 0.38 | 32.1 | 34.7 | 1.59 | 1.97 | 0 | 0 | 321 | 347 |
| 5 | 4.2 | 4.7 | 2.04 | 2.03 | 0.34 | 0.55 | 37.6 | 47.4 | 1.92 | 1.94 | 0 | 0 | 364 | 348 |
| 6 | 11.5 | 1.1 | 2.08 | 2.02 | 0.12 | 0.20 | 34.4 | 34.9 | 1.93 | 1.95 | 1 | 0 | 355 | 328 |
| 7 | 1.5 | 3.9 | 2.03 | 2.20 | 0.29 | 0.58 | 32.9 | 34.3 | 1.89 | 1.90 | 0 | 0 | 337 | 322 |
| 8 | 8.1 | 2.8 | 2.11 | 2.07 | 0.52 | 0.13 | 37.6 | 30.8 | 1.92 | 1.99 | 0 | 0 | 333 | 377 |
| 9 | 8.5 | 4.4 | 2.03 | 2.17 | 0.46 | 0.21 | 37.8 | 42.4 | 1.90 | 1.96 | 0 | 0 | 356 | 329 |
| 10 | 6.8 | 8.5 | 2.14 | 2.11 | 0.20 | 0.32 | 40.0 | 42.6 | 1.90 | 1.92 | 0 | 0 | 326 | 331 |
| 11 | 0.7 | 5.5 | 2.27 | 2.06 | 0.03 | 0.45 | 42.7 | 32.7 | 1.82 | 1.97 | 0 | 1 | 217 | 379 |
| 12 | 7.0 | 1.6 | 2.06 | 2.12 | 0.25 | 0.43 | 33.1 | 33.0 | 1.93 | 1.94 | 0 | 0 | 372 | 329 |
| 13 | 5.9 | 7.0 | 2.06 | 2.06 | 0.06 | 0.37 | 38.4 | 40.7 | 1.97 | 1.85 | 0 | 0 | 373 | 335 |
| 14 | 9.9 | 6.3 | 2.11 | 2.04 | 0.46 | 0.36 | 34.2 | 33.1 | 1.84 | 1.94 | 0 | 1 | 322 | 378 |
| 15 | 2.8 | 8.6 | 2.25 | 2.07 | 0.16 | 0.21 | 34.1 | 30.8 | 1.86 | 1.93 | 0 | 0 | 298 | 378 |
| 16 | 8.9 | 6.4 | 2.15 | 2.25 | 0.33 | 0.39 | 38.6 | 39.9 | 1.94 | 1.90 | 0 | 0 | 328 | 290 |
| 17 | 5.1 | 7.1 | 2.15 | 2.08 | 0.41 | 0.42 | 32.5 | 31.2 | 1.87 | 1.86 | 0 | 0 | 334 | 357 |
| 18 | 5.0 | 7.4 | 2.10 | 2.04 | 0.35 | 0.43 | 35.7 | 34.8 | 1.93 | 1.76 | 0 | 0 | 354 | 338 |
| 19 | 7.0 | 9.4 | 2.26 | 2.04 | 0.29 | 0.32 | 32.0 | 33.2 | 1.94 | 1.77 | 0 | 0 | 319 | 344 |
| 20 | 4.0 | 1.7 | 2.07 | 2.04 | 0.22 | 0.53 | 34.4 | 46.8 | 1.90 | 1.98 | 0 | 0 | 363 | 332 |
| 21 | 7.1 | 4.1 | 2.12 | 2.11 | 0.17 | 0.11 | 33.9 | 41.2 | 1.94 | 1.76 | 0 | 0 | 357 | 311 |
| 22 | 4.8 | 5.8 | 2.03 | 2.08 | 0.39 | 0.18 | 37.4 | 34.9 | 1.93 | 1.96 | 0 | 0 | 367 | 373 |
| 23 | 7.7 | 5.4 | 2.11 | 2.11 | 0.21 | 0.16 | 36.7 | 36.6 | 1.95 | 1.79 | 0 | 0 | 354 | 330 |
| 24 | 8.4 | 7.9 | 2.10 | 2.05 | 0.33 | 0.36 | 35.6 | 35.5 | 1.99 | 1.96 | 0 | 0 | 360 | 374 |
| 25 | 5.8 | 4.2 | 2.01 | 2.09 | 0.48 | 0.05 | 44.6 | 33.6 | 1.85 | 1.81 | 0 | 0 | 341 | 349 |
| 26 | 1.7 | 5.9 | 2.01 | 2.15 | 0.53 | 0.18 | 36.2 | 32.2 | 1.84 | 1.97 | 0 | 0 | 338 | 364 |
| 27 | 8.0 | 11.1 | 2.30 | 2.08 | 0.37 | 0.24 | 39.3 | 32.1 | 1.87 | 1.96 | 0 | 1 | 260 | 362 |
| 28 | 5.1 | 3.9 | 2.07 | 2.12 | 0.27 | 0.16 | 30.5 | 35.8 | 1.87 | 1.94 | 0 | 0 | 366 | 357 |
| 29 | 4.9 | 6.4 | 2.08 | 2.13 | 0.07 | 0.05 | 37.1 | 36.8 | 1.95 | 1.90 | 0 | 1 | 366 | 350 |
| 30 | 5.6 | 6.8 | 2.01 | 2.00 | 0.36 | 0.20 | 44.3 | 37.5 | 1.89 | 1.91 | 0 | 0 | 354 | 378 |

Table 12 Optimization results for the CVT method over 30 independent runs

| NO | X1 | X2 | X3 | X4 | X5 | X6 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CVT | CVT | CVT | CVT | CVT | CVT | CVT |
| 1 | 6.2 | 2.09 | 0.31 | 34.5 | 1.86 | 0 | 345 |
| 2 | 6.3 | 2.01 | 0.08 | 36.4 | 1.91 | 0 | 381 |
| 3 | 3.1 | 2.11 | 0.36 | 35.0 | 1.92 | 0 | 345 |
| 4 | 15.6 | 2.19 | 0.37 | 33.5 | 1.91 | 1 | 278 |
| 5 | 7.4 | 2.06 | 0.44 | 38.7 | 1.97 | 0 | 360 |
| 6 | 2.7 | 2.16 | 0.63 | 34.3 | 1.74 | 0 | 300 |
| 7 | 5.7 | 2.10 | 0.62 | 38.0 | 1.95 | 0 | 351 |
| 8 | 10.9 | 2.12 | 0.50 | 32.3 | 1.98 | 1 | 332 |
| 9 | 2.7 | 2.07 | 0.92 | 39.3 | 1.81 | 0 | 326 |
| 10 | 6.7 | 2.18 | 0.71 | 33.0 | 1.90 | 0 | 327 |
| 11 | 7.3 | 2.07 | 0.58 | 32.3 | 1.91 | 0 | 362 |
| 12 | 5.8 | 2.07 | 0.31 | 33.4 | 1.93 | 0 | 371 |
| 13 | 0.6 | 2.29 | 0.48 | 30.6 | 1.95 | 0 | 248 |
| 14 | 8.8 | 2.26 | 0.62 | 31.4 | 1.79 | 0 | 279 |
| 15 | 6.6 | 2.14 | 0.45 | 38.3 | 1.90 | 0 | 329 |
| 16 | 9.4 | 2.20 | 0.40 | 34.8 | 1.99 | 0 | 333 |
| 17 | 8.9 | 2.06 | 0.44 | 36.1 | 1.84 | 0 | 343 |
| 18 | 15.8 | 2.09 | 0.34 | 36.2 | 1.72 | 1 | 269 |
| 19 | 11.9 | 2.17 | 0.63 | 40.7 | 2.00 | 1 | 290 |
| 20 | 8.3 | 2.17 | 0.33 | 38.4 | 1.92 | 0 | 322 |
| 21 | 8.6 | 2.14 | 0.66 | 36.2 | 1.96 | 0 | 338 |
| 22 | 8.8 | 2.12 | 0.56 | 30.1 | 1.92 | 0 | 353 |
| 23 | 8.3 | 2.11 | 0.46 | 38.2 | 1.78 | 0 | 306 |
| 24 | 5.0 | 2.08 | 0.20 | 40.6 | 1.83 | 0 | 334 |
| 25 | 3.6 | 2.15 | 0.41 | 37.2 | 1.76 | 0 | 317 |
| 26 | 8.1 | 2.07 | 0.25 | 35.1 | 1.94 | 0 | 368 |
| 27 | 12.4 | 2.14 | 0.84 | 35.1 | 1.86 | 1 | 273 |
| 28 | 6.9 | 2.03 | 0.33 | 36.1 | 1.90 | 0 | 369 |
| 29 | 8.8 | 2.11 | 0.38 | 37.4 | 1.87 | 0 | 329 |
| 30 | 9.2 | 2.16 | 0.33 | 33.0 | 1.93 | 0 | 342 |

Table 13 Statistical analysis of quality loss (QL) and maximum flight height (MH)

| Method | Mean |  | Median |  | Max |  | Min |  | Q1 |  | Q3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL |
| PM | 365.6 | 1908.09 | 373 | 1456.03 | 391 | 4798.06 | 303 | 91.75 | 348.25 | 1128.42 | 381.75 | 2967.72 |
| LHS | 351.6 | 4496.26 | 356 | 4166.58 | 379 | 13731.95 | 287 | 1279.43 | 347.25 | 2767.04 | 364.25 | 5655.17 |
| CVT | 327.33 | 8549.5 | 332.5 | 8538.47 | 381 | 15838.97 | 248 | 1719.21 | 308.75 | 5774.53 | 349.5 | 11256.48 |
| D_opt | 338.27 | 6099.77 | 344.5 | 5423.04 | 373 | 20689.3 | 217 | 1076.5 | 326.5 | 4158.41 | 359.25 | 6954.43 |
| G_opt | 345.9 | 5288.1 | 347.5 | 4660.24 | 379 | 16077.51 | 275 | 271.08 | 330.25 | 3033.79 | 370.75 | 7422.89 |

(a) maximum flight height                                 （b）quality loss

Figure 10 Box plots of quality loss and maximum flight height obtained from 30 independent optimization runs

Figure 11 Flight trajectories optimized by five different methods

The box plots in Fig. 10 for the maximum flight height (MH) and quality loss (QL) metrics vividly illustrate the performance disparities among the five methods. Compared with the alternatives, the proposed PM method exhibits a superior distribution for the MH metric, characterized by a higher median and mean. Furthermore, its more compact interquartile range and fewer outliers indicate greater result concentration and enhanced stability. For the QL metric, the PM method achieves a significantly lower quality loss, with a lower median, a lower mean, and a more concentrated distribution, as evidenced by a narrower interquartile range and fewer, smaller-valued outliers. These results demonstrate that the PM method identifies optimal solutions that are both higher in performance and more robust.

A detailed quantitative analysis based on the data in Tables 10–13 confirms that the PM method substantially outperforms the next-best method, LHS. For the QL metric, PM reduces the mean QL from 4496.26 to 1908.09, a 57.56% decrease. The improvement in median QL is even more pronounced at 65.05%. This superiority extends to the extremes of the distribution: PM slashes the maximum QL by 65.06% (from 13731.95 to 4798.06) and achieves a remarkable 92.83% reduction in the minimum QL (from 1279.43 to 91.75). For the MH metric, PM also surpasses LHS across the board, increasing the mean by 3.98%, the median by 4.78%, and the maximum and minimum values by 3.17% and 5.57%, respectively. This demonstrates its comprehensive efficacy in co-optimizing both objectives.

The performance advantage of the PM method is even more stark when compared with the other methods. In contrast to CVT, which had the highest mean QL, PM achieves a 77.68% reduction in mean QL (1908.09 vs. 8549.5) and an 82.95% reduction in median QL. It also demonstrates superior control over extreme values, lowering the maximum QL by 69.71% and the minimum QL by 94.66% (from 1719.21 to 91.75). Similarly, PM outperforms D-optimality, which produced the single worst-case outcome with a maximum QL of 20689.3; PM’s maximum QL is 76.81% lower, significantly enhancing system robustness. The mean QL is also 68.72% lower than that of D-optimality. For the MH metric, PM’s mean is 8.08% higher, and its minimum MH (303) is 39.63% greater than the lowest value recorded by any method (217, from D-optimality). Finally, compared with G-optimality, PM reduces the mean and median QL by 63.92% and 68.76%, respectively. Even though G-optimality yielded the second-best minimum QL (271.08), PM improves upon this by a further 66.16%, showcasing its superior search capability. The improvements in MH, while consistent, are of a smaller magnitude than those for QL. This is attributable to the relative sparsity of the newly added design points within the high-dimensional, complex input-output space. Nevertheless, the consistent superiority in both reducing quality loss and increasing flight height clearly indicates that the proposed method identifies more effective and robust optimal solutions. Therefore, the proposed modeling approach, which integrates an attention mechanism, can rapidly search and accurately delineate the latent subspace of optimal solutions, providing a powerful tool for non-stationary response surface modeling and parameter optimization in data-scarce scenarios.

As depicted by the orange curve in Fig. 11, the flight trajectory optimized by the PM method reaches its apex of 390 m at 7.5 s. The trajectory shows a rapid ascent followed by a near-linear descent, returning to the ground at approximately 122 s. While the other methods reach their peak altitudes at similar times, their peaks are substantially lower. Although the ascent profiles are comparable, the PM trajectory achieves a higher apex and exhibits a slower descent, indicating superior overall flight performance. This suggests that the attention mechanism effectively prioritizes data points within the optimal region during response surface modeling, thereby enhancing local model accuracy. This improved accuracy translates into better manufacturing parameter settings and, consequently, enhanced rocket performance. In summary, the PM method outperforms the LHS method in both flight altitude and duration, making it highly suitable for mission scenarios with demanding performance requirements.

## 4.5 Discussion

This study introduces an experimental design algorithm that builds upon an initial set of training samples by incorporating an attention-based weighting mechanism. At its core, the algorithm employs an attention model to dynamically focus on the potential subregion of the optimal solution, which guides the preferential allocation of new design points to this critical area. This strategy markedly improves the predictive accuracy within the key subregion, thereby enhancing the robustness and reliability of the final optimization outcome.

While this approach intentionally trades a degree of global predictive accuracy for enhanced local modeling fidelity, its overall effectiveness is not compromised. This is for two reasons. First, the addition of new design points, even when concentrated in the optimal subregion, still contributes to the overall predictive accuracy of the response surface model. Second, for robust parameter design problems, the predictive fidelity within the neighborhood of the optimum is the critical determinant of the final optimization quality. Therefore, concentrating computational resources and experimental efforts on this specific region is paramount.

The iterative, attention-guided update mechanism of the proposed method (Algorithm 1) incurs a higher computational cost than classical one-shot modeling approaches, and its overhead is comparable to that of other adaptive sampling techniques. However, the key advantage of this approach is its efficiency in application: for the ultimate goal of robust parameter optimization, the resulting model is far more effective than those produced by comparable methods.

# 5 Conclusion

To address the challenge of insufficient predictive accuracy in non-stationary response surface models derived from small datasets, this study introduces a method for non-stationary Gaussian Process (GP) modeling and robust parameter optimization that incorporates an attention-based weighting mechanism. Initially, a treed modeling approach is employed to construct a non-stationary GP response surface, accurately capturing the complex relationship between inputs and outputs. Subsequently, a two-stage attention-weighting mechanism is implemented. This mechanism, which integrates weights from a quality loss function and a maximin distance criterion, dynamically focuses on the latent subregion of the optimal solution. This focus guides the allocation of new design points, creating a locally high-fidelity model that is highly efficient for robust parameter optimization. Finally, a genetic algorithm is utilized to determine the optimal parameter settings.

The core advantage of the proposed method lies in its attention-guided sampling strategy, which rapidly identifies and concentrates on the optimal subregion of the non-stationary response. By focusing experimental resources in this manner, the predictive accuracy within this critical area is substantially improved. As the fidelity of this subregion directly dictates the overall performance of the robust optimal solution, this approach significantly enhances the optimization efficiency of non-stationary GP models under small-sample conditions and improves the robustness and reliability of the final solution.

It is important to note that this method employs a focused attention strategy, prioritizing predictive accuracy in the latent optimal subregion to maximize efficiency in robust parameter optimization tasks. However, this strategy does not explicitly target global model accuracy, a characteristic that may limit its applicability to other types of optimization problems. Future work will therefore explore the development of attention mechanisms that balance both global and local predictive accuracy to broaden the applicability of this approach.

# References

[1]	He Z, Hu H, Liu H L, Wang W. A review on the theoretical research and application of lean Six Sigma[J]. Industrial Engineering Journal, 2021, 24(05): 1–8.

[2]	Ma Y Z. Theory and methods of reducing and controlling variation of muitivariate quality characteristics[D]. Xian: Northwestern Polytechnical University, 2002.

[3]	Wang J J, Ma Y Z, Ouyang L H, Sun J S, Liu J. Bayesian modeling and optimization of multi-response robust parameter design[J]. Journal of Management Sciences in China, 2016, 19(02): 85–94.

[4]	Feng Z B, Wang J J, Ma Y Z. Bayesian modeling and robust parameter design based on multivariate Gaussian process model[J]. Systems Engineering — Theory & Practice, 2020, 40(03): 703–713.

[5]	Williams C. Gaussian Processes for Machine Learning[J].

[6]	Santner T J, Williams B J, Notz W I. Space-Filling Designs for Computer Experiments[M]//Santner T J, Williams B J, Notz W I. The Design and Analysis of Computer Experiments. New York, NY: Springer New York, 2018: 145-200.

[7]	Tan M H Y, Wu C F J. Robust Design Optimization With Quadratic Loss Derived From Gaussian Process Models[J]. Technometrics, 2012, 54(1): 51-63.

[8]	Sun L, Hong L J, Hu Z. Balancing Exploitation and Exploration in Discrete Optimization via Simulation Through a Gaussian Process-Based Search[J]. Operations Research, 2014, 62(6): 1416-1438.

[9]	Bogunovic I, Scarlett J, Jegelka S, et al. Adversarially robust optimization with Gaussian processes[J]. Advances in neural information processing systems, 2018, 31.

[10]	Zhang L, Wang K, Chen N. Monitoring wafers’ geometric quality using an additive Gaussian process model[J]. IIE Transactions, 2016, 48(1): 1-15.

[11]	Booth A S, Cooper A, Gramacy R B. Nonstationary Gaussian Process Surrogates[A]. arXiv, 2024.

[12]	Wang G D, Li X Y. Planning and analysis of sequential accelerated life tests under model uncertainty[J]. Systems Engineering — Theory & Practice, 2024: 1–21.

[13]	Cui Q A, Zhu A Q. Research on Robust Parameter Design of 3D Printing Based on GPR [J]. Industrial Engineering and Management, 2022, 27(05): 127-135.

[14]	Gramacy R B, Lee H K H. Bayesian Treed Gaussian Process Models With an Application to Computer Modeling[J]. Journal of the American Statistical Association, 2008, 103(483): 1119-1130.

[15]	Konomi B, Karagiannis G, Sarkar A, et al. Bayesian Treed Multivariate Gaussian Process With Adaptive Design: Application to a Carbon Capture Unit[J]. Technometrics, 2014, 56(2): 145-158.

[16]	Pandita P, Tsilifis P, Awalgaonkar N M, et al. Surrogate-based sequential Bayesian experimental design using non-stationary Gaussian Processes[J]. Computer Methods in Applied Mechanics and Engineering, 2021, 385: 114007.

[17]	Paun I, Husmeier D, Torney C J. Stochastic variational inference for scalable non-stationary Gaussian process regression[J]. Statistics and Computing, 2023, 33(2): 44.

[18]	Antunes F, Amorim M, Pereira F C, et al. Active learning metamodeling for policy analysis: Application to an emergency medical service simulator[J]. Simulation Modelling Practice and Theory, 2019, 97: 101947.

[19]	Meng Z, Zhang Z, Zhou H, et al. Robust design optimization of imperfect stiffened shells using an active learning method and a hybrid surrogate model[J]. Engineering Optimization, 2020, 52(12): 2044-2061.

[20]	Wang Z, Broccardo M. A novel active learning-based Gaussian process metamodelling strategy for estimating the full probability distribution in forward UQ analysis[J]. Structural Safety, 2020, 84: 101937.

[21]	Han M, Huang Q, Ouyang L, et al. A kriging-based active learning algorithm for contour estimation of integrated response with noise factors[J]. Engineering with Computers, 2023, 39(2): 1341-1362.

[22]	Niu Z, Zhong G, Yu H. A review on the attention mechanism of deep learning[J]. Neurocomputing, 2021, 452: 48-62.

[23]	Vaswani A, Shazeer N, Parmar N, et al. Attention is All you Need[C]//Advances in Neural Information Processing Systems: Vol. 30. Curran Associates, Inc., 2017.

[24]	Li T, Ma J. Attention mechanism based mixture of gaussian processes[J]. Pattern Recognition Letters, 2022, 161: 130-136.

[25]	Xie J, Ma Z, Chang D, et al. GPCA: A probabilistic framework for gaussian process embedded channel attention[J]. IEEE Transactions on Pattern Analysis and Machine Intelligence, 2021: 1-1.

[26]	Zhong Y, Ling Z, Liu L, et al. Deep Gaussian Attention Network for Lumber Surface Defect Segmentation[J]. IEEE Transactions on Instrumentation and Measurement, 2024, 73: 1-12.

[27]	Huynh S K, Trieu N M, Tran D H. Attention-Driven with Gaussian Processes for Weakly Supervised Hemorrhage Detection in Brain CT Scans[M]//Zhang Q, Henry C, Jensen R, et al. Rough Sets: Vol. 15710. Cham: Springer Nature Switzerland, 2025: 272-286.

[28]	Taguchi G, Taguchi G, Elsayed E A, et al.. Quality engineering in production systems[M]. McGraw-Hill Companies, 1989.

[29]	Wanyonyi S W, Okango A A, Koech J K. Exploration of D-, A-, I-and G-Optimality Criteria in Mixture Modeling[J]. 2021.

[30]	Guo B, Li X R, Liu M Q, et al. Construction of orthogonal general sliced Latin hypercube designs[J]. Statistical Papers, 2023, 64(3): 987-1014.

[31]	Du Q, Faber V, Gunzburger M. Centroidal Voronoi Tessellations: Applications and Algorithms[J]. SIAM Review, 1999, 41(4): 637-676.

[32]	He H, Xiong R, Guo H. Online estimation of model parameters and state-of-charge of LiFePO4 batteries in electric vehicles[J]. Applied Energy, 2012, 89(1): 413-420.

[33]	Shrivastava P, Tey K S. Battery Test Data (LiFePO4 18650 Rechargeable Cell 3.3V 1100 mAh, Panasonic NCR18650B 3400mAh, Murata VTC6 18650 3000mAh 15A)[DS/OL]. Mendeley Data, 2022. https://data.mendeley.com/datasets/29kw38kzwj/1.

[34]	Mustafa H, Bourelly C, Vitelli M, et al. SoC estimation on Li-ion batteries: A new EIS-based dataset for data-driven applications[J]. Data in Brief, 2024, 57: 110947.

# Appendix A

The modeling procedure for the treed non-stationary Gaussian Process (GP) is detailed as follows. First, the input space is recursively partitioned into R non-overlapping regions . Each region contains a dataset comprising observations. Let denote the number of covariates in the data matrix plus an intercept term. For each region , the hierarchical generative GP model is defined as:

where , and  is an matrix. The distributions , , and denote the (multivariate) Normal, Inverse Gamma, and Wishart distributions, respectively. The hyperparameters are assumed to be known. Equation (33) specifies a multivariate normal likelihood with linear trend coefficients , variance, and an  correlation matrix . The coefficients are assumed to be drawn from a common but unknown mean with a region-specific variance . The model formulation lacks an explicit mechanism to ensure continuity of the process across the boundaries of adjacent regions. However, smoothness can be captured through model averaging, and thus the ability to fit discontinuous surfaces is considered a feature of this model.

Subsequently, the GP correlation structure for each partition is defined as , where is derived from an isotropic or separable power-exponential family with a fixed power , but with unknown (random) range and nugget parameters. As the subsequent discussion is largely applicable to generated by other families and for an unfixed , the correlation parameters are referred to indirectly via the resulting correlation matrix or kernel function .

Finally, a Gamma mixture prior is specified for the hyperparameter :

This prior assigns approximately equal weight to small values of , representing a population of GP parameterizations for highly variable surfaces, while also establishing a separate population of parameters for surfaces that are substantially smoother or nearly linear. An exponential distribution,, is adopted as the prior for . Alternatively, the prior can be expressed as , with a reference prior subsequently used for .

For hyperparameter estimation, the dataset is used to update the GP parameters for each region . Given a tree T, the set of all parameters is denoted by , where represents the top-level parameters from the hierarchical prior, which also require updating. A Markov Chain Monte Carlo (MCMC) algorithm is used to sample from the posterior distribution of . This is implemented as a block Gibbs sampler, which first draws from the conditional posterior of the region-specific parameters,, and then draws from the conditional posterior of the top-level parameters,. All hyperparameters are updated via Gibbs sampling, with the exception of the parameters of the covariance function , such as , which are updated using a Metropolis-Hastings Algorithm.

# Appendix B

Table 14 Comparison of maximum response values for the 10 attention-guided design samples

| Total sample size | Max |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.1976 | 0.1426 | 0.2912 | 0.1105 | 0.1619 |
| 50 | 0.1206 | 0.0990 | 0.0935 | 0.0777 | 0.0840 |
| 60 | 0.0871 | 0.0824 | 0.0679 | 0.1891 | 0.0544 |
| 70 | 0.0826 | 0.0801 | 0.0815 | 0.0784 | 0.0490 |
| 80 | 0.0581 | 0.1178 | 0.0539 | 0.0565 | 0.0440 |
| 90 | 0.0566 | 0.0465 | 0.0409 | 0.0618 | 0.0480 |
| 100 | 0.0313 | 0.0371 | 0.0365 | 0.0510 | 0.0290 |

Table 15 Comparison of minimum response values for the 10 attention-guided design samples

| Total sample size | Min |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.0398 | 0.0639 | 0.0265 | 0.0381 | 0.0213 |
| 50 | 0.0143 | 0.0232 | 0.0260 | 0.0167 | 0.0126 |
| 60 | 0.0302 | 0.0064 | 0.0051 | 0.0332 | 0.0210 |
| 70 | 0.0012 | 0.0025 | 0.0234 | 0.0029 | 0.0047 |
| 80 | 0.0191 | 0.0008 | 0.0212 | 0.0208 | 0.0134 |
| 90 | 0.0017 | 0.0204 | 0.0048 | 0.0167 | 0.0122 |
| 100 | 0.0003 | 0.0167 | 0.0067 | 0.0016 | 0.0004 |

Table 16 Comparison of Q1 values for the 10 attention-guided design samples.

| Total sample size | Q1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.0665 | 0.0729 | 0.0587 | 0.0490 | 0.0381 |
| 50 | 0.0468 | 0.0542 | 0.0488 | 0.0522 | 0.0322 |
| 60 | 0.0425 | 0.0481 | 0.0371 | 0.0390 | 0.0319 |
| 70 | 0.0380 | 0.0372 | 0.0308 | 0.0298 | 0.0274 |
| 80 | 0.0308 | 0.0315 | 0.0281 | 0.0263 | 0.0212 |
| 90 | 0.0253 | 0.0275 | 0.0231 | 0.0231 | 0.0177 |
| 100 | 0.0210 | 0.0217 | 0.0188 | 0.0211 | 0.0153 |

Table 17 Comparison of Q3 values for the 10 attention-guided design samples.

| Total sample size | Q3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.1066 | 0.1070 | 0.0908 | 0.0794 | 0.0597 |
| 50 | 0.0741 | 0.0721 | 0.0686 | 0.0661 | 0.0441 |
| 60 | 0.0578 | 0.0604 | 0.0538 | 0.0585 | 0.0420 |
| 70 | 0.0483 | 0.0523 | 0.0442 | 0.0408 | 0.0358 |
| 80 | 0.0415 | 0.0452 | 0.0363 | 0.0394 | 0.0285 |
| 90 | 0.0369 | 0.0357 | 0.0314 | 0.0302 | 0.0238 |
| 100 | 0.0270 | 0.0274 | 0.0268 | 0.0283 | 0.0222 |

Analysis of the data in Tables 14–17 provides insights into model performance based on three key metrics: maximum RMSE, minimum RMSE, and interquartile range (IQR).

Worst-Case Performance (Maximum RMSE): The maximum RMSE reflects the upper bound of model error across independent experiments. The PM method demonstrates a robust capability in minimizing this worst-case error, an advantage that becomes increasingly pronounced as the training sample size grows. With a sample size of 40, the PM method's maximum RMSE was 0.1619, representing a 44.40% reduction in worst-case error compared to the poorest-performing LHS method (0.2912). The superiority of the PM method is further highlighted at larger sample sizes. At a sample size of 100, the PM method achieved the lowest maximum RMSE of all methods (0.0290). This constitutes an improvement of approximately 7.35% over the second-best method, D-Optimality (0.0313), and a substantial 43.14% improvement over the worst-performing CVT method (0.0510). Overall, the PM method exhibits a significant suppressive effect on maximum error, with the most substantial improvement—71.23%—observed against the CVT method with 60 training samples.

Best-Case Performance (Minimum RMSE): The minimum RMSE reveals the optimal performance potentially achievable by the model. The PM method also excels in this regard, consistently yielding the lowest or near-lowest minimum RMSE values, which indicates a high probability of identifying high-quality solutions. With 40 samples, the PM method achieved the lowest minimum RMSE (0.0213), outperforming all other techniques. This represents a 19.62% enhancement in best-case performance over the competitive LHS method (0.0265) and a 66.67% enhancement over the G-Optimality method (0.0639). This capacity for finding optimal solutions is maintained at larger sample sizes. At 100 samples, the PM method's minimum RMSE of 0.0004 was nearly on par with that of the D-Optimality method (0.0003), with both methods leading the field. In contrast, its performance was approximately 97.60% superior to the G-Optimality method, which had the highest minimum RMSE (0.0167). Although other methods occasionally achieved marginally lower minimums at specific sample sizes (e.g., 70 and 90), the PM method consistently demonstrated top-tier potential for optimal performance.

Performance Robustness (Interquartile Range): The interquartile range (IQR = Q3 − Q1), calculated from the first (Q1) and third (Q3) quartiles, allows for an evaluation of the robustness and stability of each method's performance; a smaller IQR signifies more concentrated and reliable results. The PM method exhibits exceptional robustness. With a sample size of 40, its IQR of 0.0216 (0.0597 − 0.0381) was the smallest among all methods. This indicates a result concentration approximately 28.95% higher than the next-best CVT method (IQR = 0.0304) and 46.13% higher than the least robust D-Optimality method (IQR = 0.0401), demonstrating the high reproducibility of the PM method's performance even with small sample sizes. As the sample size increased to 100, the robustness of all methods improved. The PM method's IQR decreased to 0.0069 (0.0222 − 0.0153), maintaining an excellent level of stability and significantly outperforming LHS (IQR = 0.0080). The most pronounced advantage in robustness was observed with 50 training samples, where the PM method's IQR of 0.0119 was 56.41% lower than that of the least robust D-Optimality method (0.0273). This provides compelling evidence that the PM method is not only superior in performance but also highly stable and reliable.

Table 18 Comparison of maximum response values for the 20 attention-guided design samples

| Total sample size | Max |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.3059 | 0.2248 | 0.1732 | 0.1170 | 0.2559 |
| 50 | 0.1864 | 0.0895 | 0.1043 | 0.1075 | 0.2452 |
| 60 | 0.1259 | 0.0999 | 0.0664 | 0.0899 | 0.0404 |
| 70 | 0.0758 | 0.0623 | 0.0574 | 0.0707 | 0.0442 |
| 80 | 0.0750 | 0.0621 | 0.0617 | 0.0670 | 0.0388 |
| 90 | 0.0538 | 0.0481 | 0.0468 | 0.0609 | 0.0512 |
| 100 | 0.0416 | 0.0485 | 0.0462 | 0.0601 | 0.0390 |

Table 19 Comparison of minimum response values for the 20 attention-guided design samples

| Total sample size | Min |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.0614 | 0.0402 | 0.0287 | 0.0559 | 0.0086 |
| 50 | 0.0336 | 0.0058 | 0.0063 | 0.0364 | 0.0009 |
| 60 | 0.0048 | 0.0232 | 0.0254 | 0.0039 | 0.0081 |
| 70 | 0.0067 | 0.0257 | 0.0129 | 0.0309 | 0.0071 |
| 80 | 0.0003 | 0.0187 | 0.0187 | 0.0044 | 0.0048 |
| 90 | 0.0147 | 0.0144 | 0.0180 | 0.0189 | 0.0002 |
| 100 | 0.0147 | 0.0145 | 0.0073 | 0.0205 | 0.0008 |

Table 20 Comparison of Q1 values for the 20 attention-guided design samples

| Total sample size | Q1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.1081 | 0.0765 | 0.0482 | 0.0741 | 0.0203 |
| 50 | 0.0701 | 0.0449 | 0.0401 | 0.0635 | 0.0185 |
| 60 | 0.0337 | 0.0404 | 0.0291 | 0.0505 | 0.0162 |
| 70 | 0.0341 | 0.0326 | 0.0269 | 0.0431 | 0.0166 |
| 80 | 0.0261 | 0.0308 | 0.0305 | 0.0369 | 0.0154 |
| 90 | 0.0240 | 0.0228 | 0.0214 | 0.0340 | 0.0135 |
| 100 | 0.0232 | 0.0285 | 0.0182 | 0.0266 | 0.0121 |

Table 21 Comparison of Q3 values for the 20 attention-guided design samples

| Total sample size | Q3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | LHS | CVT | PM |
| 40 | 0.1775 | 0.1003 | 0.0861 | 0.0883 | 0.0584 |
| 50 | 0.1237 | 0.0599 | 0.0665 | 0.0782 | 0.0297 |
| 60 | 0.0611 | 0.0553 | 0.0457 | 0.0639 | 0.0242 |
| 70 | 0.0538 | 0.0463 | 0.0438 | 0.0600 | 0.0270 |
| 80 | 0.0418 | 0.0514 | 0.0413 | 0.0482 | 0.0302 |
| 90 | 0.0395 | 0.0346 | 0.0299 | 0.0443 | 0.0230 |
| 100 | 0.0323 | 0.0358 | 0.0271 | 0.0388 | 0.0193 |

An analysis of the data presented in Tables 18–21 for the 20-design-sample case reveals the consistent superiority of the PM method across various performance metrics.

Worst-Case Performance (Maximum RMSE): The PM method excels at controlling worst-case error. With 60 training samples, its maximum RMSE dropped to 0.0404, the lowest among all methods. This represents a substantial 67.91% reduction in error compared to the poorest-performing D-Optimality method (0.1259). The PM method subsequently maintained the lowest maximum RMSE at sample sizes of 70, 80, and 100. Notably, at 100 samples, its maximum RMSE of 0.0390 was 6.25% lower than the next-best method, D-Optimality (0.0416), and 35.11% lower than the worst-performing CVT method (0.0601). These results indicate that the PM method's ability to mitigate extreme errors becomes increasingly stable and effective as data volume grows. The greatest relative improvement (67.91%) was observed against D-Optimality at 60 samples, while the smallest (6.25%) was also against D-Optimality, but at 100 samples.

Best-Case Performance (Minimum RMSE): The PM method demonstrates a commanding advantage in identifying optimal solutions. Across the majority of training sample sizes, it achieved the lowest or near-lowest minimum RMSE. With 40 samples, its minimum RMSE of 0.0086 was significantly better than all competitors, marking an 86.00% enhancement in optimal performance over the D-Optimality method (0.0614). This advantage peaked at 50 samples, where the PM method's minimum RMSE was just 0.0009. At 90 samples, its minimum RMSE of 0.0002 far surpassed all other methods. These findings underscore the PM method's substantial and stable potential for exploring the solution space and discovering high-quality models, consistently identifying top-performing solutions regardless of sample size. The maximum performance gain was approximately 98.64% over the D-Optimality method at 90 samples, while the minimum gain was still a remarkable 69.00% over the LHS method at 40 samples.

Performance Robustness (Interquartile Range): An analysis of the interquartile range (IQR = Q3 − Q1) confirms the exceptional stability of the PM method. With 50 training samples, the PM method's IQR of 0.0112 was the smallest recorded, indicating a 79.10% greater concentration of results compared to the least robust D-Optimality method (IQR = 0.0536). The PM method also maintained the smallest IQR at sample sizes of 60 and 70; for instance, at 60 samples, its IQR of 0.0080 was approximately 70.80% smaller than that of the most unstable D-Optimality method (0.0274). At 100 samples, its IQR of 0.0072 was tied with the G-Optimality method (0.0073) for the best performance. The most significant improvement in robustness (79.10%) was observed against D-Optimality at 50 samples, and the smallest (1.37%) was against G-Optimality at 100 samples. This further substantiates that the PM method maintains highly stable performance across diverse conditions.

# Appendix C

Table 22 Optimization results from 30 independent runs for the LHS and PM methods using 60 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM |
| 1 | 4.6 | 7.9 | 2.12 | 2.04 | 0.38 | 0.87 | 32.6 | 34.8 | 1.81 | 2.00 | 0 | 0 | 333 | 374 |
| 2 | 4.1 | 3.8 | 2.07 | 2.04 | 0.30 | 0.69 | 37.1 | 32.4 | 1.90 | 1.99 | 0 | 0 | 354 | 383 |
| 3 | 6.5 | 5.1 | 2.11 | 2.19 | 0.32 | 0.26 | 38.1 | 31.2 | 1.89 | 1.97 | 0 | 0 | 336 | 350 |
| 4 | 6.5 | 6.9 | 2.08 | 2.03 | 0.40 | 0.31 | 31.6 | 30.9 | 1.89 | 2.00 | 0 | 0 | 358 | 392 |
| 5 | 7.3 | 5.8 | 2.09 | 2.02 | 0.19 | 0.22 | 37.0 | 37.5 | 1.90 | 1.98 | 0 | 0 | 352 | 383 |
| 6 | 7.2 | 3.9 | 2.10 | 2.01 | 0.19 | 0.20 | 35.2 | 33.2 | 1.85 | 2.00 | 0 | 0 | 341 | 392 |
| 7 | 6.6 | 3.9 | 2.08 | 2.12 | 0.49 | 0.48 | 34.8 | 33.6 | 1.89 | 1.87 | 0 | 0 | 349 | 341 |
| 8 | 4.4 | 4.6 | 2.07 | 2.04 | 0.56 | 0.12 | 30.5 | 38.8 | 1.87 | 1.94 | 0 | 0 | 360 | 370 |
| 9 | 4.8 | 3.8 | 2.14 | 2.05 | 0.03 | 0.31 | 33.6 | 34.9 | 1.88 | 1.95 | 0 | 0 | 347 | 371 |
| 10 | 5.6 | 2.8 | 2.06 | 2.10 | 0.31 | 0.54 | 34.5 | 31.9 | 1.86 | 1.89 | 0 | 0 | 357 | 350 |
| 11 | 2.3 | 10.8 | 2.04 | 2.05 | 0.05 | 0.77 | 32.8 | 39.1 | 1.88 | 2.00 | 0 | 1 | 358 | 333 |
| 12 | 7.0 | 2.0 | 2.07 | 2.03 | 0.34 | 0.95 | 33.0 | 32.3 | 1.89 | 1.94 | 0 | 0 | 359 | 359 |
| 13 | 7.4 | 1.9 | 2.13 | 2.04 | 0.51 | 0.49 | 31.9 | 31.9 | 1.79 | 1.97 | 0 | 0 | 320 | 360 |
| 14 | 5.1 | 12.0 | 2.06 | 2.02 | 0.30 | 0.53 | 35.8 | 31.9 | 1.98 | 1.94 | 0 | 1 | 374 | 346 |
| 15 | 7.0 | 2.7 | 2.08 | 2.01 | 0.31 | 0.53 | 36.5 | 31.6 | 1.89 | 1.99 | 0 | 0 | 349 | 383 |
| 16 | 5.2 | 8.5 | 2.22 | 2.02 | 0.39 | 0.25 | 32.5 | 32.3 | 1.90 | 1.99 | 0 | 0 | 322 | 391 |
| 17 | 6.0 | 2.7 | 2.04 | 2.18 | 0.31 | 0.29 | 36.4 | 31.3 | 1.95 | 1.96 | 0 | 0 | 373 | 340 |
| 18 | 7.2 | 5.5 | 2.02 | 2.01 | 0.36 | 0.16 | 33.3 | 39.6 | 1.85 | 1.88 | 0 | 0 | 365 | 367 |
| 19 | 7.3 | 1.5 | 2.03 | 2.01 | 0.42 | 0.54 | 34.9 | 35.2 | 1.88 | 1.95 | 0 | 0 | 360 | 346 |
| 20 | 5.8 | 7.0 | 2.07 | 2.08 | 0.22 | 0.28 | 35.0 | 33.1 | 1.85 | 1.97 | 0 | 0 | 351 | 372 |
| 21 | 7.7 | 6.7 | 2.08 | 2.04 | 0.45 | 0.11 | 34.6 | 34.3 | 1.85 | 1.96 | 0 | 0 | 339 | 384 |
| 22 | 5.2 | 6.7 | 2.04 | 2.03 | 0.28 | 0.26 | 35.9 | 32.8 | 1.83 | 1.97 | 0 | 0 | 354 | 386 |
| 23 | 7.2 | 2.2 | 2.05 | 2.07 | 0.23 | 0.52 | 35.8 | 30.8 | 1.86 | 1.99 | 0 | 0 | 357 | 363 |
| 24 | 4.1 | 3.9 | 2.05 | 2.06 | 0.35 | 0.14 | 35.2 | 32.1 | 1.90 | 1.95 | 0 | 0 | 364 | 379 |
| 25 | 7.5 | 4.5 | 2.08 | 2.03 | 0.41 | 0.28 | 35.0 | 39.9 | 1.91 | 1.97 | 0 | 0 | 353 | 373 |
| 26 | 5.0 | 6.1 | 2.09 | 2.00 | 0.55 | 0.25 | 34.1 | 33.5 | 1.88 | 1.93 | 0 | 0 | 348 | 387 |
| 27 | 4.8 | 9.5 | 2.12 | 2.04 | 0.24 | 0.94 | 35.8 | 33.2 | 1.91 | 1.84 | 0 | 0 | 347 | 344 |
| 28 | 6.2 | 6.1 | 2.07 | 2.05 | 0.38 | 0.59 | 33.2 | 30.9 | 1.89 | 1.95 | 0 | 0 | 359 | 378 |
| 29 | 9.0 | 14.1 | 2.02 | 2.15 | 0.08 | 0.57 | 35.8 | 31.8 | 1.89 | 2.00 | 0 | 1 | 377 | 300 |
| 30 | 4.9 | 3.8 | 2.17 | 2.05 | 0.12 | 0.47 | 36.6 | 31.4 | 1.94 | 1.95 | 0 | 0 | 340 | 378 |

Table 23 Optimization results from 30 independent runs for the G_opt and D_opt methods using 60 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt |
| 1 | 10.9 | 10.8 | 2.07 | 2.03 | 0.62 | 0.66 | 33.2 | 31.6 | 1.93 | 1.90 | 1 | 1 | 324 | 335 |
| 2 | 3.1 | 5.9 | 2.09 | 2.01 | 0.83 | 0.57 | 33.5 | 34.0 | 1.95 | 1.86 | 0 | 0 | 356 | 368 |
| 3 | 5.3 | 7.3 | 2.17 | 2.06 | 0.33 | 0.18 | 40.5 | 38.4 | 1.84 | 1.79 | 0 | 0 | 299 | 338 |
| 4 | 7.3 | 13.9 | 2.12 | 2.26 | 0.45 | 0.55 | 36.8 | 37.2 | 1.90 | 1.99 | 0 | 1 | 333 | 256 |
| 5 | 8.4 | 11.1 | 2.18 | 2.11 | 0.11 | 0.85 | 41.2 | 34.1 | 1.89 | 1.86 | 0 | 1 | 314 | 290 |
| 6 | 7.7 | 6.6 | 2.04 | 2.01 | 0.49 | 0.40 | 37.5 | 39.8 | 1.85 | 1.75 | 0 | 0 | 344 | 331 |
| 7 | 7.8 | 2.2 | 2.21 | 2.03 | 0.35 | 0.43 | 37.3 | 38.8 | 1.88 | 1.87 | 0 | 0 | 299 | 343 |
| 8 | 9.1 | 11.2 | 2.09 | 2.28 | 0.45 | 0.91 | 35.6 | 31.8 | 1.98 | 1.93 | 0 | 1 | 358 | 251 |
| 9 | 9.0 | 11.2 | 2.05 | 2.07 | 0.66 | 0.52 | 33.3 | 35.5 | 1.89 | 1.89 | 0 | 1 | 351 | 319 |
| 10 | 5.2 | 3.5 | 2.02 | 2.01 | 0.86 | 0.23 | 44.4 | 31.3 | 1.86 | 1.97 | 0 | 0 | 339 | 391 |
| 11 | 9.3 | 6.2 | 2.09 | 2.01 | 0.12 | 0.18 | 35.7 | 39.4 | 1.93 | 1.90 | 0 | 0 | 362 | 372 |
| 12 | 11.2 | 11.2 | 2.28 | 2.17 | 0.71 | 0.62 | 30.4 | 35.6 | 1.82 | 1.83 | 1 | 1 | 239 | 265 |
| 13 | 6.6 | 8.5 | 2.04 | 2.04 | 0.17 | 0.12 | 31.6 | 31.9 | 1.94 | 1.91 | 0 | 0 | 388 | 384 |
| 14 | 7.0 | 5.8 | 2.04 | 2.17 | 0.58 | 0.58 | 31.5 | 33.3 | 1.84 | 1.87 | 0 | 0 | 363 | 325 |
| 15 | 0.5 | 13.6 | 2.06 | 2.01 | 0.08 | 0.55 | 33.4 | 42.0 | 1.88 | 1.98 | 0 | 1 | 279 | 336 |
| 16 | 7.0 | 11.3 | 2.10 | 2.05 | 0.71 | 0.01 | 33.3 | 38.6 | 1.94 | 1.95 | 0 | 1 | 374 | 373 |
| 17 | 5.3 | 5.5 | 2.25 | 2.17 | 0.22 | 0.43 | 40.2 | 36.0 | 2.00 | 1.97 | 0 | 0 | 318 | 343 |
| 18 | 1.6 | 5.6 | 2.19 | 2.03 | 0.13 | 0.35 | 32.9 | 38.4 | 1.54 | 1.86 | 0 | 0 | 252 | 354 |
| 19 | 4.4 | 7.8 | 2.20 | 2.07 | 0.09 | 0.07 | 32.9 | 36.3 | 1.96 | 1.84 | 0 | 0 | 347 | 357 |
| 20 | 6.1 | 6.5 | 2.12 | 2.04 | 0.27 | 0.38 | 31.8 | 40.5 | 1.83 | 1.98 | 0 | 0 | 345 | 367 |
| 21 | 7.0 | 10.5 | 2.09 | 2.23 | 0.44 | 0.23 | 32.5 | 32.8 | 1.84 | 1.96 | 0 | 1 | 347 | 316 |
| 22 | 6.5 | 4.8 | 2.01 | 2.06 | 0.85 | 0.41 | 35.6 | 39.0 | 1.85 | 1.85 | 0 | 0 | 360 | 339 |
| 23 | 6.1 | 9.9 | 2.08 | 2.12 | 0.59 | 0.28 | 30.0 | 35.2 | 1.76 | 1.86 | 0 | 0 | 342 | 328 |
| 24 | 13.7 | 7.2 | 2.07 | 2.03 | 0.50 | 0.16 | 36.1 | 32.4 | 1.84 | 1.94 | 1 | 0 | 293 | 386 |
| 25 | 6.6 | 12.4 | 2.01 | 2.11 | 0.43 | 0.17 | 33.1 | 30.8 | 1.78 | 1.94 | 0 | 1 | 360 | 351 |
| 26 | 18.1 | 6.3 | 2.03 | 2.09 | 0.98 | 0.29 | 31.6 | 36.3 | 2.00 | 1.68 | 1 | 0 | 308 | 302 |
| 27 | 0.4 | 8.2 | 2.08 | 2.00 | 0.31 | 0.42 | 40.6 | 36.9 | 1.99 | 1.76 | 0 | 0 | 270 | 338 |
| 28 | 6.7 | 3.8 | 2.15 | 2.16 | 0.49 | 0.38 | 32.1 | 41.4 | 1.84 | 1.84 | 0 | 0 | 329 | 298 |
| 29 | 8.4 | 4.6 | 2.09 | 2.09 | 0.20 | 0.08 | 34.9 | 40.1 | 1.91 | 1.92 | 0 | 0 | 359 | 353 |
| 30 | 7.2 | 2.9 | 2.23 | 2.08 | 0.39 | 0.70 | 37.7 | 39.0 | 1.87 | 1.80 | 0 | 0 | 294 | 319 |

Table 24 Optimization results from 30 independent runs for the CVT methods using 60 training samples

| NO | X1 | X2 | X3 | X4 | X5 | X6 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CVT | CVT | CVT | CVT | CVT | CVT | CVT |
| 1 | 10.8 | 2.15 | 0.18 | 32.0 | 1.92 | 1 | 339 |
| 2 | 8.4 | 2.30 | 0.37 | 38.3 | 1.91 | 0 | 270 |
| 3 | 2.2 | 2.09 | 0.53 | 41.4 | 1.92 | 0 | 328 |
| 4 | 10.1 | 2.24 | 0.45 | 37.2 | 1.91 | 1 | 271 |
| 5 | 9.7 | 2.10 | 0.37 | 33.1 | 1.77 | 0 | 317 |
| 6 | 2.2 | 2.34 | 0.53 | 36.8 | 1.94 | 0 | 269 |
| 7 | 5.4 | 2.10 | 0.34 | 37.1 | 1.98 | 0 | 360 |
| 8 | 0.3 | 2.22 | 0.97 | 33.4 | 1.95 | 0 | 246 |
| 9 | 13.5 | 2.05 | 0.90 | 37.0 | 1.98 | 1 | 317 |
| 10 | 0.4 | 2.02 | 0.49 | 41.4 | 1.78 | 0 | 262 |
| 11 | 6.2 | 2.14 | 0.44 | 35.1 | 1.94 | 0 | 342 |
| 12 | 2.3 | 2.16 | 0.82 | 31.4 | 1.75 | 0 | 304 |
| 13 | 6.5 | 2.05 | 0.53 | 37.8 | 1.86 | 0 | 344 |
| 14 | 6.3 | 2.14 | 0.31 | 35.1 | 1.78 | 0 | 313 |
| 15 | 2.2 | 2.15 | 0.49 | 35.4 | 1.81 | 0 | 305 |
| 16 | 14.6 | 2.06 | 0.05 | 36.9 | 1.90 | 1 | 357 |
| 17 | 8.0 | 2.07 | 0.32 | 37.0 | 1.78 | 0 | 325 |
| 18 | 6.8 | 2.08 | 0.58 | 30.6 | 1.85 | 0 | 349 |
| 19 | 6.5 | 2.10 | 0.27 | 31.7 | 1.93 | 0 | 365 |
| 20 | 3.7 | 2.04 | 0.07 | 37.2 | 1.88 | 0 | 362 |
| 21 | 10.2 | 2.08 | 0.07 | 34.7 | 1.93 | 1 | 365 |
| 22 | 5.2 | 2.16 | 0.32 | 33.3 | 1.92 | 0 | 340 |
| 23 | 9.4 | 2.12 | 0.55 | 33.3 | 1.96 | 0 | 347 |
| 24 | 8.0 | 2.09 | 0.36 | 36.9 | 1.98 | 0 | 360 |
| 25 | 7.2 | 2.07 | 0.43 | 32.0 | 1.87 | 0 | 356 |
| 26 | 7.8 | 2.13 | 0.68 | 38.2 | 1.83 | 0 | 305 |
| 27 | 10.1 | 2.10 | 0.25 | 32.1 | 1.91 | 1 | 346 |
| 28 | 8.6 | 2.17 | 0.88 | 33.1 | 1.89 | 0 | 316 |
| 29 | 6.0 | 2.09 | 0.38 | 37.6 | 1.95 | 0 | 355 |
| 30 | 4.1 | 2.09 | 0.13 | 33.4 | 1.81 | 0 | 344 |

Table 25 Comparison of statistical metrics from experiments with 60 training samples

| Method | Mean |  | Median |  | Max |  | Min |  | Q1 |  | Q3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL |
| PM | 365.83 | 2930.53 | 371.5 | 3013.73 | 392 | 6720.97 | 300 | 1018.22 | 350 | 2160.4 | 383 | 3456.65 |
| LHS | 351.87 | 5784.58 | 353.5 | 5342.49 | 377 | 9498.79 | 320 | 2690.94 | 347 | 4459.98 | 359 | 7648.59 |
| CVT | 325.97 | 8622.45 | 339.5 | 7396.28 | 365 | 21212.57 | 246 | 777.46 | 307 | 6151.12 | 353.5 | 10521.37 |
| D_opt | 328.2 | 7570.85 | 340.5 | 6076.41 | 388 | 19800.33 | 239 | 1474 | 301.25 | 4308.73 | 357.5 | 9276.37 |
| G_opt | 334.27 | 7235.88 | 338 | 6161.14 | 391 | 17281.67 | 251 | 1879.86 | 319 | 4146.96 | 356.25 | 9606.4 |

The data presented in the table reveals the superior performance of the PM method compared to four established design methodologies across two key metrics: Quality Loss (QL), which is to be minimized, and the MH metric, which is to be maximized.

Comparison with LHS: When benchmarked against the next-best method, LHS, the PM method demonstrates significant advantages. For the QL metric, PM reduced the mean QL by 49.34% (from 5784.58 to 2930.53) and the median QL by 43.59%. Critically, it lowered the minimum QL by 62.16% (from 2690.94 to 1018.22). For the MH metric, PM outperformed LHS across all statistical measures, achieving improvements of 3.97% in the mean, 5.09% in the median, and 3.98% in the maximum value. This indicates a comprehensive superiority in co-optimizing both objectives.

Comparison with CVT: The performance gap between PM and CVT is even more pronounced. CVT exhibited the poorest performance on the QL metric. In contrast, the PM method's mean QL of 2930.53 represents a 66.01% reduction from CVT's 8622.45. Similarly, PM's median QL was 59.25% lower. In terms of mitigating extreme values, PM reduced the maximum QL by 68.32% (from 21212.57 to 6720.97), substantially enhancing system stability. For the MH metric, PM's mean value (365.83) was 12.23% higher than that of CVT (325.97). Furthermore, PM's minimum MH value (300) was 21.95% higher than CVT's (246), effectively raising the performance floor.

Comparison with D-optimality: The PM method outperformed the D-optimality design across all dimensions. D-optimality produced the second-highest maximum QL (19800.33), whereas PM's maximum QL was 66.06% lower. The mean QL for PM was 61.29% lower than that for D-optimality (7570.85). The improvements in the MH metric were also considerable, with PM showing an 11.47% higher mean and a 9.10% higher median. D-optimality yielded the lowest minimum MH value (239) among all methods; PM's minimum MH of 300 was 25.52% higher, demonstrating a decisive advantage in guaranteeing worst-case performance.

Comparison with G-optimality: Against the G-optimality design, which was relatively weak on the MH metric, PM again showed superior results. For the QL metric, PM's mean and median values were 59.50% and 51.08% lower than those of G-optimality (mean QL of 7235.88), respectively. At the distributional tails, PM reduced the maximum QL by 61.10% and the minimum QL by 45.84%. For the MH metric, PM's mean and median values were 9.44% and 9.91% higher than G-optimality's. Its minimum MH value (300) represents a 19.52% improvement over G-optimality's minimum (251), further highlighting the robustness and exceptional performance of the PM method in non-stationary optimization.

Table 26 Optimization results from 30 independent runs for the LHS and PM methods using 70 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM |
| 1 | 3.1 | 1.1 | 2.32 | 2.05 | 0.58 | 0.28 | 33.9 | 32.0 | 1.91 | 1.94 | 0 | 0 | 284 | 318 |
| 2 | 5.0 | 6.5 | 2.08 | 2.03 | 0.32 | 0.12 | 35.0 | 32.1 | 1.87 | 1.95 | 0 | 0 | 351 | 390 |
| 3 | 4.0 | 7.0 | 2.00 | 2.03 | 0.53 | 0.18 | 31.7 | 37.2 | 1.92 | 2.00 | 0 | 0 | 384 | 383 |
| 4 | 8.1 | 3.9 | 2.06 | 2.03 | 0.43 | 0.36 | 34.9 | 34.3 | 1.83 | 1.96 | 0 | 0 | 339 | 378 |
| 5 | 4.9 | 7.2 | 2.04 | 2.00 | 0.18 | 0.10 | 35.5 | 33.1 | 1.81 | 1.93 | 0 | 0 | 353 | 392 |
| 6 | 4.1 | 3.6 | 2.04 | 2.06 | 0.28 | 0.57 | 31.8 | 30.5 | 1.94 | 1.97 | 0 | 0 | 381 | 379 |
| 7 | 8.1 | 7.2 | 2.08 | 2.04 | 0.41 | 0.12 | 34.2 | 32.0 | 1.82 | 1.88 | 0 | 0 | 334 | 375 |
| 8 | 8.3 | 2.6 | 2.12 | 2.05 | 0.03 | 0.18 | 36.4 | 31.2 | 1.85 | 1.93 | 0 | 0 | 343 | 369 |
| 9 | 6.8 | 5.8 | 2.14 | 2.04 | 0.32 | 0.16 | 38.8 | 34.7 | 1.92 | 1.95 | 0 | 0 | 331 | 381 |
| 10 | 1.7 | 7.3 | 2.08 | 2.12 | 0.54 | 0.18 | 34.2 | 30.6 | 1.81 | 1.85 | 0 | 0 | 318 | 351 |
| 11 | 2.8 | 5.5 | 2.08 | 2.07 | 0.61 | 0.08 | 33.6 | 38.7 | 1.90 | 2.00 | 0 | 0 | 351 | 374 |
| 12 | 4.1 | 3.8 | 2.06 | 2.20 | 0.16 | 0.07 | 31.8 | 30.4 | 1.90 | 1.92 | 0 | 0 | 372 | 343 |
| 13 | 6.6 | 5.6 | 2.07 | 2.03 | 0.32 | 0.06 | 33.6 | 37.2 | 1.89 | 1.79 | 0 | 0 | 360 | 352 |
| 14 | 6.7 | 4.2 | 2.08 | 2.03 | 0.24 | 0.15 | 33.9 | 38.4 | 1.89 | 1.98 | 0 | 0 | 357 | 377 |
| 15 | 0.1 | 6.0 | 2.18 | 2.05 | 0.09 | 0.08 | 34.3 | 36.6 | 1.89 | 1.96 | 0 | 0 | 242 | 378 |
| 16 | 7.8 | 7.5 | 2.03 | 2.02 | 0.34 | 0.10 | 32.2 | 34.3 | 1.91 | 1.92 | 0 | 0 | 375 | 384 |
| 17 | 3.3 | 2.1 | 2.05 | 2.02 | 0.33 | 0.07 | 31.9 | 32.6 | 1.94 | 1.92 | 0 | 0 | 373 | 365 |
| 18 | 2.9 | 1.9 | 2.22 | 2.06 | 0.68 | 0.38 | 32.8 | 32.2 | 1.92 | 1.95 | 0 | 0 | 316 | 352 |
| 19 | 7.9 | 9.4 | 2.08 | 2.08 | 0.24 | 0.09 | 36.7 | 37.6 | 1.93 | 1.88 | 0 | 0 | 358 | 352 |
| 20 | 8.8 | 5.5 | 2.10 | 2.02 | 0.31 | 0.16 | 36.8 | 39.3 | 1.87 | 1.98 | 0 | 0 | 334 | 381 |
| 21 | 6.8 | 7.6 | 2.14 | 2.08 | 0.14 | 0.17 | 41.2 | 32.9 | 1.94 | 1.98 | 0 | 0 | 336 | 378 |
| 22 | 4.3 | 3.8 | 2.03 | 2.02 | 0.52 | 0.18 | 32.9 | 32.3 | 1.84 | 1.93 | 0 | 0 | 363 | 383 |
| 23 | 5.8 | 5.7 | 2.07 | 2.01 | 0.17 | 0.23 | 35.4 | 39.7 | 1.86 | 1.94 | 0 | 0 | 357 | 374 |
| 24 | 9.3 | 5.7 | 2.03 | 2.06 | 0.27 | 0.15 | 34.0 | 33.5 | 1.81 | 1.99 | 0 | 0 | 353 | 383 |
| 25 | 8.1 | 2.4 | 2.06 | 2.09 | 0.36 | 0.43 | 30.9 | 30.2 | 1.90 | 1.95 | 0 | 0 | 365 | 359 |
| 26 | 5.1 | 3.0 | 2.13 | 2.06 | 0.23 | 0.08 | 33.9 | 35.1 | 1.88 | 1.95 | 0 | 0 | 344 | 366 |
| 27 | 7.5 | 7.0 | 2.02 | 2.08 | 0.40 | 0.13 | 34.2 | 38.3 | 1.82 | 1.92 | 0 | 0 | 355 | 357 |
| 28 | 3.6 | 5.0 | 2.00 | 2.03 | 0.16 | 0.11 | 35.8 | 40.0 | 1.96 | 1.97 | 0 | 0 | 384 | 376 |
| 29 | 6.9 | 2.3 | 2.12 | 2.03 | 0.42 | 0.10 | 36.5 | 32.8 | 1.87 | 1.98 | 0 | 0 | 328 | 371 |
| 30 | 5.4 | 6.6 | 2.11 | 2.11 | 0.40 | 0.43 | 33.2 | 30.6 | 1.84 | 1.96 | 0 | 0 | 339 | 365 |

Table 27 Optimization results from 30 independent runs for the G_opt and D_opt methods using 70 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt |
| 1 | 9.1 | 8.4 | 2.11 | 2.11 | 0.33 | 0.69 | 36.2 | 32.7 | 1.88 | 1.86 | 0 | 0 | 333 | 331 |
| 2 | 3.4 | 3.7 | 2.08 | 2.20 | 0.21 | 0.41 | 37.5 | 41.0 | 1.92 | 1.82 | 0 | 0 | 354 | 282 |
| 3 | 6.9 | 7.1 | 2.02 | 2.07 | 0.25 | 0.31 | 38.3 | 47.1 | 1.88 | 1.72 | 0 | 0 | 363 | 280 |
| 4 | 4.0 | 8.3 | 2.15 | 2.05 | 0.09 | 0.40 | 41.8 | 33.4 | 1.96 | 1.95 | 0 | 0 | 334 | 369 |
| 5 | 7.6 | 0.8 | 2.11 | 2.13 | 0.41 | 0.31 | 31.0 | 35.0 | 1.94 | 1.93 | 0 | 0 | 358 | 281 |
| 6 | 6.5 | 5.5 | 2.16 | 2.05 | 0.23 | 0.54 | 33.3 | 36.9 | 1.89 | 1.98 | 0 | 0 | 338 | 371 |
| 7 | 2.7 | 5.7 | 2.12 | 2.03 | 0.40 | 0.40 | 44.0 | 44.7 | 1.54 | 1.99 | 0 | 0 | 233 | 366 |
| 8 | 7.6 | 5.1 | 2.02 | 2.32 | 0.27 | 0.53 | 32.3 | 31.8 | 1.88 | 1.81 | 0 | 0 | 373 | 275 |
| 9 | 5.4 | 5.5 | 2.11 | 2.03 | 0.40 | 0.45 | 33.6 | 34.6 | 1.81 | 1.97 | 0 | 0 | 330 | 381 |
| 10 | 6.6 | 4.4 | 2.06 | 2.14 | 0.51 | 0.02 | 35.9 | 38.6 | 1.84 | 1.96 | 0 | 0 | 342 | 349 |
| 11 | 7.9 | 2.5 | 2.11 | 2.14 | 0.22 | 0.60 | 40.2 | 34.3 | 1.96 | 1.90 | 0 | 0 | 347 | 329 |
| 12 | 8.2 | 5.8 | 2.12 | 2.02 | 0.52 | 0.19 | 37.8 | 31.9 | 1.86 | 1.92 | 0 | 0 | 316 | 387 |
| 13 | 8.6 | 1.2 | 2.03 | 2.15 | 0.13 | 0.47 | 33.6 | 32.3 | 1.79 | 1.81 | 0 | 0 | 357 | 289 |
| 14 | 8.5 | 3.9 | 2.02 | 2.06 | 0.32 | 0.21 | 44.9 | 41.1 | 1.89 | 1.99 | 0 | 0 | 343 | 363 |
| 15 | 7.2 | 10.9 | 2.10 | 2.06 | 0.28 | 0.38 | 39.8 | 34.6 | 1.93 | 1.92 | 0 | 1 | 342 | 339 |
| 16 | 8.4 | 5.4 | 2.29 | 2.09 | 0.12 | 0.12 | 31.3 | 36.7 | 1.89 | 1.99 | 0 | 0 | 308 | 371 |
| 17 | 7.3 | 3.0 | 2.07 | 2.26 | 0.22 | 0.02 | 35.7 | 33.1 | 1.96 | 1.85 | 0 | 0 | 370 | 303 |
| 18 | 3.5 | 8.1 | 2.10 | 2.21 | 0.43 | 0.07 | 41.1 | 34.4 | 1.72 | 1.88 | 0 | 0 | 292 | 325 |
| 19 | 8.7 | 4.7 | 2.02 | 2.04 | 0.30 | 0.38 | 32.9 | 34.1 | 1.91 | 1.82 | 0 | 0 | 374 | 354 |
| 20 | 9.7 | 6.6 | 2.09 | 2.13 | 0.15 | 0.25 | 41.4 | 35.4 | 1.90 | 1.97 | 0 | 0 | 339 | 355 |
| 21 | 7.8 | 5.1 | 2.29 | 2.20 | 0.36 | 0.27 | 37.1 | 30.2 | 1.96 | 1.89 | 0 | 0 | 292 | 335 |
| 22 | 8.9 | 3.7 | 2.11 | 2.06 | 0.05 | 0.23 | 41.6 | 38.6 | 1.88 | 1.98 | 0 | 0 | 336 | 367 |
| 23 | 10.2 | 1.4 | 2.11 | 2.07 | 0.37 | 0.02 | 33.1 | 33.2 | 1.79 | 1.97 | 1 | 0 | 302 | 332 |
| 24 | 4.2 | 7.6 | 2.05 | 2.18 | 0.40 | 0.57 | 34.6 | 36.3 | 1.94 | 1.96 | 0 | 0 | 369 | 325 |
| 25 | 8.0 | 12.4 | 2.22 | 2.02 | 0.36 | 0.69 | 36.2 | 31.9 | 1.99 | 1.81 | 0 | 1 | 323 | 310 |
| 26 | 5.8 | 6.8 | 2.03 | 2.18 | 0.72 | 0.50 | 33.6 | 32.8 | 1.68 | 1.92 | 0 | 0 | 326 | 330 |
| 27 | 2.6 | 4.1 | 2.09 | 2.04 | 0.14 | 0.29 | 44.5 | 43.6 | 1.94 | 1.84 | 0 | 0 | 332 | 335 |
| 28 | 7.9 | 6.2 | 2.02 | 2.08 | 0.47 | 0.31 | 34.4 | 37.2 | 1.78 | 1.88 | 0 | 0 | 342 | 345 |
| 29 | 4.9 | 10.1 | 2.03 | 2.09 | 0.36 | 0.50 | 35.7 | 34.8 | 1.93 | 1.75 | 0 | 1 | 372 | 289 |
| 30 | 5.0 | 8.8 | 2.13 | 2.04 | 0.61 | 0.74 | 34.9 | 31.9 | 1.81 | 1.86 | 0 | 0 | 320 | 354 |

Table 28 Optimization results from 30 independent runs for the CVT methods using 70 training samples

| NO | X1 | X2 | X3 | X4 | X5 | X6 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CVT | CVT | CVT | CVT | CVT | CVT | CVT |
| 1 | 7.5 | 2.11 | 0.37 | 33.1 | 1.88 | 0 | 344 |
| 2 | 11.7 | 2.03 | 0.13 | 36.2 | 1.85 | 1 | 351 |
| 3 | 2.7 | 2.12 | 0.22 | 33.5 | 1.94 | 0 | 348 |
| 4 | 6.8 | 2.10 | 0.16 | 36.0 | 1.88 | 0 | 347 |
| 5 | 11.4 | 2.11 | 0.25 | 32.8 | 1.98 | 1 | 349 |
| 6 | 7.8 | 2.16 | 0.09 | 35.5 | 1.89 | 0 | 337 |
| 7 | 4.8 | 2.10 | 0.36 | 35.1 | 1.93 | 0 | 354 |
| 8 | 8.2 | 2.10 | 0.43 | 39.1 | 1.91 | 0 | 334 |
| 9 | 11.8 | 2.07 | 0.31 | 35.5 | 1.87 | 1 | 326 |
| 10 | 3.7 | 2.04 | 0.20 | 37.6 | 1.84 | 0 | 351 |
| 11 | 6.1 | 2.11 | 0.17 | 30.0 | 1.90 | 0 | 366 |
| 12 | 6.0 | 2.12 | 0.19 | 34.5 | 1.82 | 0 | 334 |
| 13 | 5.2 | 2.26 | 0.88 | 34.5 | 1.99 | 0 | 315 |
| 14 | 9.9 | 2.10 | 0.12 | 33.0 | 1.68 | 0 | 316 |
| 15 | 6.7 | 2.01 | 0.53 | 34.6 | 1.86 | 0 | 363 |
| 16 | 0.8 | 2.08 | 0.63 | 37.3 | 1.90 | 0 | 287 |
| 17 | 9.9 | 2.14 | 0.14 | 39.9 | 1.92 | 0 | 332 |
| 18 | 4.8 | 2.23 | 0.42 | 32.3 | 1.99 | 0 | 333 |
| 19 | 6.3 | 2.23 | 0.01 | 43.0 | 1.75 | 0 | 261 |
| 20 | 8.7 | 2.01 | 0.45 | 38.0 | 1.80 | 0 | 337 |
| 21 | 9.6 | 2.12 | 0.40 | 39.0 | 1.75 | 0 | 285 |
| 22 | 1.6 | 2.15 | 0.44 | 30.9 | 1.95 | 0 | 321 |
| 23 | 6.0 | 2.01 | 0.36 | 34.1 | 1.95 | 0 | 385 |
| 24 | 5.2 | 2.05 | 0.33 | 36.1 | 1.93 | 0 | 367 |
| 25 | 0.9 | 2.16 | 0.27 | 37.8 | 1.91 | 0 | 273 |
| 26 | 7.1 | 2.05 | 0.26 | 39.4 | 1.94 | 0 | 362 |
| 27 | 4.1 | 2.02 | 0.15 | 41.5 | 1.93 | 0 | 365 |
| 28 | 5.8 | 2.14 | 0.64 | 35.4 | 1.94 | 0 | 339 |
| 29 | 4.2 | 2.10 | 0.59 | 39.9 | 1.98 | 0 | 349 |
| 30 | 3.6 | 2.02 | 0.29 | 37.7 | 1.82 | 0 | 351 |

Table 29 Comparison of statistical metrics from experiments with 70 training samples

| Method | Mean |  | Median |  | Max |  | Min |  | Q1 |  | Q3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL |
| PM | 369.53 | 3527.62 | 374.5 | 2863.82 | 392 | 8610.52 | 318 | 1854.14 | 360.5 | 2516.13 | 380.5 | 4044.5 |
| LHS | 346 | 7056.32 | 352 | 6489.23 | 384 | 19136.57 | 242 | 1914.78 | 334.5 | 4823.5 | 362.25 | 8758.49 |
| CVT | 336.07 | 8476.93 | 341.5 | 7220.5 | 385 | 26473.52 | 261 | 2497.03 | 327.5 | 5729.68 | 351 | 9735.58 |
| D_opt | 335.33 | 8640.93 | 338.5 | 7532.22 | 374 | 25676.47 | 233 | 3308.62 | 323.75 | 4919.09 | 356.25 | 10550.84 |
| G_opt | 334.07 | 7835.69 | 335 | 5989.16 | 387 | 18779.07 | 275 | 2748.34 | 313.75 | 4785.09 | 361 | 10509.83 |

The PM method again demonstrated a substantial performance advantage over the other design methodologies.

Comparison with LHS: Against the next-best performing method, LHS, PM reduced the mean Quality Loss (QL) by 50.00% (from 7056.32 to 3527.62) and the median QL by 55.88% (from 6489.23 to 2863.82). This advantage was particularly pronounced in mitigating tail risk, as PM lowered the maximum QL by 55.00% (from 19136.57 to 8610.52). For the MH metric, PM improved the mean and median values by 6.80% and 6.39%, respectively. Notably, it raised the performance floor (minimum MH value) by 31.40%, from 242 to 318.

Comparison with CVT: The CVT method yielded poor QL results, with a high mean (8476.93) and maximum (26473.52). In contrast, the PM method achieved a 58.38% lower mean QL and a 67.48% reduction in the maximum QL. For the MH metric, PM's superiority was also evident, with its mean value (369.53) being 9.96% higher than CVT's (336.07). Furthermore, PM's minimum MH value (318) surpassed CVT's (261) by 21.84%, underscoring its effectiveness in establishing a high minimum performance standard.

Comparison with D-optimality: The D-optimality design produced the highest mean QL (8640.93) among all methods. The PM method's mean QL was 59.18% lower. In terms of controlling extreme values, PM reduced the maximum QL by 66.46% relative to D-optimality. For the MH metric, PM's mean value was 10.20% higher. D-optimality registered the lowest minimum MH value (233) of all methods, whereas PM's minimum of 318 was 36.48% higher.

Comparison with G-optimality: When compared to the G-optimality design, PM reduced the mean and median QL by 54.98% and 52.18%, respectively, from G-optimality's mean of 7835.69. For the MH metric, PM's mean and median values were 10.61% and 11.79% higher. Moreover, PM's minimum MH value (318) was 15.64% higher than that of G-optimality (275). These results reaffirm the PM method's comprehensive superiority in simultaneously optimizing for quality loss and performance while ensuring system robustness.

Table 30 Optimization results from 30 independent runs for the LHS and PM methods using 80 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM | LHS | PM |
| 1 | 7.8 | 1.5 | 2.08 | 2.01 | 0.21 | 0.29 | 35.0 | 32.4 | 1.84 | 1.93 | 0 | 0 | 346 | 347 |
| 2 | 5.4 | 5.0 | 2.07 | 2.10 | 0.27 | 0.10 | 36.3 | 32.1 | 1.91 | 1.88 | 0 | 0 | 361 | 363 |
| 3 | 5.4 | 5.6 | 2.16 | 2.08 | 0.20 | 0.33 | 34.9 | 33.8 | 1.93 | 2.00 | 0 | 0 | 342 | 375 |
| 4 | 1.6 | 5.8 | 2.33 | 2.01 | 0.22 | 0.24 | 33.8 | 36.3 | 1.94 | 1.95 | 0 | 0 | 272 | 382 |
| 5 | 6.5 | 4.7 | 2.21 | 2.03 | 0.23 | 0.33 | 34.9 | 35.6 | 1.86 | 1.65 | 0 | 0 | 310 | 319 |
| 6 | 7.4 | 4.3 | 2.01 | 2.06 | 0.19 | 0.04 | 33.4 | 33.1 | 1.92 | 1.97 | 0 | 0 | 384 | 383 |
| 7 | 5.9 | 3.3 | 2.06 | 2.06 | 0.29 | 0.22 | 33.3 | 31.2 | 1.96 | 1.97 | 0 | 0 | 376 | 378 |
| 8 | 6.1 | 3.4 | 2.03 | 2.04 | 0.85 | 0.09 | 36.3 | 33.2 | 1.86 | 1.96 | 0 | 0 | 352 | 380 |
| 9 | 7.7 | 4.2 | 2.12 | 2.06 | 0.32 | 0.18 | 35.9 | 33.2 | 1.95 | 1.92 | 0 | 0 | 347 | 374 |
| 10 | 5.7 | 1.2 | 2.07 | 2.07 | 0.67 | 0.08 | 39.8 | 31.2 | 1.97 | 1.95 | 0 | 0 | 356 | 325 |
| 11 | 7.8 | 4.6 | 2.07 | 2.06 | 0.01 | 0.53 | 36.8 | 31.1 | 1.90 | 1.95 | 0 | 0 | 367 | 376 |
| 12 | 4.6 | 6.0 | 2.05 | 2.08 | 0.34 | 0.30 | 42.6 | 34.2 | 1.94 | 1.87 | 0 | 0 | 356 | 355 |
| 13 | 6.3 | 2.6 | 2.10 | 2.06 | 0.06 | 0.24 | 35.4 | 32.1 | 1.86 | 1.98 | 0 | 0 | 350 | 371 |
| 14 | 2.8 | 7.2 | 2.17 | 2.12 | 0.29 | 0.23 | 37.3 | 32.1 | 1.93 | 1.97 | 0 | 0 | 323 | 367 |
| 15 | 3.8 | 8.2 | 2.06 | 2.06 | 0.38 | 0.13 | 34.5 | 33.1 | 1.97 | 1.97 | 0 | 0 | 371 | 381 |
| 16 | 3.5 | 4.3 | 2.08 | 2.06 | 0.23 | 0.13 | 33.5 | 30.1 | 1.89 | 1.97 | 0 | 0 | 359 | 388 |
| 17 | 8.7 | 4.2 | 2.02 | 2.03 | 0.47 | 0.35 | 35.8 | 34.3 | 1.90 | 1.94 | 0 | 0 | 361 | 378 |
| 18 | 6.1 | 7.3 | 2.02 | 2.08 | 0.36 | 0.18 | 35.8 | 35.2 | 1.91 | 1.99 | 0 | 0 | 372 | 376 |
| 19 | 3.6 | 5.7 | 2.14 | 2.03 | 0.24 | 0.18 | 35.7 | 33.3 | 1.90 | 1.98 | 0 | 0 | 337 | 390 |
| 20 | 8.8 | 2.6 | 2.02 | 2.15 | 0.27 | 0.04 | 35.1 | 31.8 | 1.97 | 1.93 | 0 | 0 | 381 | 345 |
| 21 | 6.1 | 4.6 | 2.04 | 2.09 | 0.13 | 0.19 | 38.1 | 35.4 | 1.77 | 1.88 | 0 | 0 | 340 | 354 |
| 22 | 7.1 | 2.0 | 2.03 | 2.06 | 0.43 | 0.07 | 34.9 | 32.6 | 1.86 | 1.99 | 0 | 0 | 359 | 359 |
| 23 | 2.8 | 7.2 | 2.08 | 2.11 | 0.31 | 0.13 | 34.9 | 33.2 | 1.96 | 1.98 | 0 | 0 | 360 | 373 |
| 24 | 5.0 | 2.9 | 2.07 | 2.08 | 0.22 | 0.06 | 35.8 | 33.1 | 1.89 | 1.97 | 0 | 0 | 356 | 368 |
| 25 | 6.8 | 6.2 | 2.11 | 2.10 | 0.13 | 0.18 | 33.9 | 32.2 | 1.90 | 1.90 | 0 | 0 | 356 | 363 |
| 26 | 5.9 | 2.4 | 2.11 | 2.03 | 0.34 | 0.17 | 36.3 | 32.5 | 1.92 | 1.98 | 0 | 0 | 349 | 372 |
| 27 | 5.7 | 2.5 | 2.10 | 2.10 | 0.40 | 0.12 | 37.8 | 32.3 | 1.85 | 1.91 | 0 | 0 | 327 | 351 |
| 28 | 4.2 | 3.8 | 2.09 | 2.09 | 0.14 | 0.23 | 35.5 | 31.4 | 1.88 | 1.75 | 0 | 0 | 353 | 339 |
| 29 | 6.9 | 7.1 | 2.05 | 2.03 | 0.35 | 0.03 | 37.4 | 32.5 | 1.95 | 1.94 | 0 | 0 | 366 | 392 |
| 30 | 5.5 | 4.0 | 2.09 | 2.07 | 0.19 | 0.12 | 39.0 | 32.4 | 1.83 | 1.94 | 0 | 0 | 331 | 374 |

Table 31 Optimization results from 30 independent runs for the G_opt and D_opt methods using 80 training samples

| NO | X1 |  | X2 |  | X3 |  | X4 |  | X5 |  | X6 |  | Y |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt | D_opt | G_opt |
| 1 | 7.6 | 5.8 | 2.14 | 2.02 | 0.51 | 0.22 | 32.8 | 33.7 | 1.76 | 1.91 | 0 | 0 | 309 | 380 |
| 2 | 4.3 | 6.2 | 2.18 | 2.19 | 0.22 | 0.06 | 33.3 | 41.7 | 1.77 | 1.94 | 0 | 0 | 310 | 322 |
| 3 | 4.4 | 9.7 | 2.08 | 2.04 | 0.24 | 0.53 | 37.0 | 34.6 | 1.89 | 1.84 | 0 | 0 | 351 | 341 |
| 4 | 8.7 | 2.6 | 2.06 | 2.08 | 0.54 | 0.51 | 32.6 | 36.1 | 1.88 | 1.89 | 0 | 0 | 352 | 342 |
| 5 | 2.6 | 0.7 | 2.07 | 2.13 | 0.49 | 0.21 | 37.1 | 33.9 | 1.93 | 1.92 | 0 | 0 | 350 | 275 |
| 6 | 6.3 | 0.2 | 2.16 | 2.16 | 0.28 | 0.58 | 32.2 | 31.6 | 1.90 | 1.78 | 0 | 0 | 341 | 241 |
| 7 | 1.6 | 5.1 | 2.17 | 2.06 | 0.04 | 0.32 | 30.5 | 30.1 | 1.88 | 1.83 | 0 | 0 | 313 | 363 |
| 8 | 9.5 | 8.9 | 2.12 | 2.07 | 0.34 | 0.10 | 38.4 | 36.5 | 1.81 | 1.89 | 0 | 0 | 305 | 361 |
| 9 | 11.0 | 4.4 | 2.18 | 2.08 | 0.69 | 0.47 | 37.9 | 32.7 | 1.89 | 1.97 | 1 | 0 | 289 | 371 |
| 10 | 6.3 | 1.6 | 2.33 | 2.23 | 0.25 | 0.12 | 39.1 | 32.5 | 1.88 | 1.81 | 0 | 0 | 263 | 284 |
| 11 | 11.5 | 4.4 | 2.02 | 2.04 | 0.53 | 0.62 | 37.3 | 34.6 | 1.87 | 1.99 | 1 | 0 | 321 | 380 |
| 12 | 2.6 | 4.6 | 2.12 | 2.06 | 0.84 | 0.59 | 33.7 | 40.0 | 1.91 | 1.86 | 0 | 0 | 339 | 338 |
| 13 | 4.9 | 2.0 | 2.09 | 2.17 | 0.34 | 0.41 | 32.3 | 35.6 | 1.92 | 1.93 | 0 | 0 | 363 | 316 |
| 14 | 9.8 | 4.9 | 2.08 | 2.08 | 0.47 | 0.32 | 38.0 | 34.6 | 1.88 | 1.96 | 0 | 0 | 329 | 368 |
| 15 | 6.5 | 6.5 | 2.03 | 2.12 | 0.58 | 0.30 | 30.5 | 38.9 | 1.77 | 1.94 | 0 | 0 | 352 | 342 |
| 16 | 0.1 | 9.7 | 2.05 | 2.01 | 0.59 | 0.51 | 46.0 | 38.0 | 1.95 | 1.88 | 0 | 0 | 252 | 352 |
| 17 | 7.7 | 4.6 | 2.08 | 2.14 | 0.39 | 0.46 | 37.3 | 32.5 | 1.89 | 1.91 | 0 | 0 | 343 | 344 |
| 18 | 6.2 | 2.3 | 2.11 | 2.17 | 0.31 | 0.22 | 35.6 | 33.7 | 1.78 | 1.93 | 0 | 0 | 319 | 327 |
| 19 | 1.0 | 10.2 | 2.02 | 2.02 | 0.43 | 0.32 | 42.3 | 38.1 | 1.72 | 1.91 | 0 | 1 | 280 | 348 |
| 20 | 10.4 | 8.3 | 2.12 | 2.01 | 0.35 | 0.51 | 35.7 | 30.9 | 1.65 | 1.62 | 1 | 0 | 264 | 325 |
| 21 | 7.4 | 4.9 | 2.33 | 2.08 | 0.15 | 0.66 | 35.3 | 36.4 | 1.84 | 1.91 | 0 | 0 | 271 | 348 |
| 22 | 1.1 | 2.6 | 2.22 | 2.09 | 0.55 | 0.18 | 39.6 | 31.1 | 1.83 | 1.78 | 0 | 0 | 252 | 337 |
| 23 | 6.7 | 6.8 | 2.08 | 2.08 | 0.64 | 0.36 | 34.0 | 35.1 | 1.78 | 1.95 | 0 | 0 | 325 | 361 |
| 24 | 14.2 | 2.3 | 2.02 | 2.13 | 0.55 | 0.51 | 32.8 | 31.7 | 1.87 | 1.70 | 1 | 0 | 315 | 305 |
| 25 | 8.7 | 6.9 | 2.10 | 2.13 | 0.48 | 0.34 | 34.6 | 33.6 | 1.83 | 1.90 | 0 | 0 | 325 | 342 |
| 26 | 4.4 | 4.9 | 2.07 | 2.12 | 0.73 | 0.29 | 38.2 | 38.4 | 1.92 | 1.77 | 0 | 0 | 349 | 308 |
| 27 | 5.9 | 0.5 | 2.16 | 2.27 | 0.34 | 0.42 | 32.4 | 32.5 | 1.98 | 1.85 | 0 | 0 | 354 | 235 |
| 28 | 7.8 | 3.1 | 2.21 | 2.05 | 0.45 | 0.06 | 32.4 | 34.4 | 1.84 | 1.85 | 0 | 0 | 305 | 356 |
| 29 | 7.3 | 4.7 | 2.07 | 2.11 | 0.34 | 0.21 | 37.1 | 39.5 | 2.00 | 1.89 | 0 | 0 | 369 | 336 |
| 30 | 7.5 | 4.3 | 2.02 | 2.02 | 0.72 | 0.64 | 36.3 | 31.9 | 1.95 | 1.99 | 0 | 0 | 372 | 389 |

Table 32 Optimization results from 30 independent runs for the CVT methods using 80 training samples

| NO | X1 | X2 | X3 | X4 | X5 | X6 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CVT | CVT | CVT | CVT | CVT | CVT | CVT |
| 1 | 2.3 | 2.13 | 0.30 | 38.5 | 1.95 | 0 | 331 |
| 2 | 1.2 | 2.07 | 0.50 | 33.8 | 1.83 | 0 | 306 |
| 3 | 7.1 | 2.06 | 0.43 | 35.1 | 1.88 | 0 | 353 |
| 4 | 7.7 | 2.12 | 0.43 | 33.7 | 1.82 | 0 | 325 |
| 5 | 8.6 | 2.18 | 0.41 | 40.2 | 1.92 | 0 | 306 |
| 6 | 10.1 | 2.22 | 0.27 | 35.2 | 1.82 | 1 | 277 |
| 7 | 9.1 | 2.05 | 0.41 | 35.4 | 1.86 | 0 | 346 |
| 8 | 3.0 | 2.09 | 0.51 | 32.7 | 1.78 | 0 | 332 |
| 9 | 6.8 | 2.11 | 0.41 | 34.9 | 1.95 | 0 | 355 |
| 10 | 4.5 | 2.13 | 0.43 | 34.5 | 1.98 | 0 | 355 |
| 11 | 3.0 | 2.12 | 0.37 | 39.6 | 1.98 | 0 | 341 |
| 12 | 6.6 | 2.24 | 0.18 | 32.5 | 1.98 | 0 | 335 |
| 13 | 5.1 | 2.03 | 0.26 | 37.1 | 1.93 | 0 | 371 |
| 14 | 7.3 | 2.03 | 0.79 | 35.3 | 1.79 | 0 | 335 |
| 15 | 7.7 | 2.18 | 0.02 | 33.7 | 1.88 | 0 | 339 |
| 16 | 10.2 | 2.00 | 0.26 | 38.4 | 1.80 | 1 | 333 |
| 17 | 5.7 | 2.11 | 0.36 | 33.6 | 1.84 | 0 | 339 |
| 18 | 4.9 | 2.06 | 0.28 | 33.8 | 1.92 | 0 | 367 |
| 19 | 7.6 | 2.10 | 0.09 | 36.7 | 1.83 | 0 | 339 |
| 20 | 7.6 | 2.06 | 0.36 | 38.1 | 1.92 | 0 | 355 |
| 21 | 12.4 | 2.05 | 0.48 | 35.4 | 1.86 | 1 | 314 |
| 22 | 3.1 | 2.13 | 0.28 | 33.4 | 1.78 | 0 | 319 |
| 23 | 8.0 | 2.08 | 0.25 | 35.6 | 1.75 | 0 | 322 |
| 24 | 4.8 | 2.23 | 0.38 | 36.8 | 1.89 | 0 | 303 |
| 25 | 1.6 | 2.01 | 0.02 | 30.1 | 1.85 | 0 | 349 |
| 26 | 1.8 | 2.18 | 0.27 | 30.8 | 1.95 | 0 | 323 |
| 27 | 6.6 | 2.18 | 0.12 | 33.6 | 1.91 | 0 | 338 |
| 28 | 6.1 | 2.14 | 0.39 | 33.8 | 1.84 | 0 | 328 |
| 29 | 5.9 | 2.02 | 0.16 | 36.4 | 1.88 | 0 | 370 |
| 30 | 8.2 | 2.06 | 0.15 | 35.3 | 1.88 | 0 | 361 |

Table 33 Comparison of statistical metrics from experiments with 80 training samples

| Method | Mean |  | Median |  | Max |  | Min |  | Q1 |  | Q3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL | MH | QL |
| PM | 366.6 | 5597.53 | 372.5 | 4794.5 | 392 | 14924.09 | 319 | 2131.16 | 356 | 3481.38 | 378 | 5943.41 |
| LHS | 350.67 | 8907.73 | 356 | 8332.6 | 384 | 21441.47 | 272 | 3271.37 | 343 | 6424 | 361 | 10717.41 |
| CVT | 335.57 | 10070.66 | 336.5 | 9343.45 | 371 | 18541.11 | 277 | 2641.21 | 323.5 | 7482.7 | 352 | 12569.66 |
| D_opt | 319.4 | 12093.48 | 323 | 10690.87 | 372 | 22646.91 | 252 | 3865.38 | 305 | 8724 | 349.75 | 16243.79 |
| G_opt | 334.57 | 9588.88 | 342 | 8141.33 | 389 | 21515.31 | 235 | 3256.61 | 322.75 | 6710.74 | 359.75 | 12136.71 |

The performance superiority of the PM method remains evident across all comparisons.

Comparison with LHS: Benchmarked against the next-best method, LHS, the PM method achieved significant enhancements on key metrics. For Quality Loss (QL), PM reduced the mean by 37.16% (from 8907.73 to 5597.53) and, more notably, the median by 42.46%. This suggests a greater ability to concentrate outcomes within the optimal solution subspace. For the MH metric, PM demonstrated comprehensive leadership, improving the mean and median values by 4.54% and 4.63%, respectively. Concurrently, it raised the performance floor (minimum MH value) by 17.28% (from 272 to 319), substantially increasing system stability and reliability.

Comparison with CVT: The PM method's superior performance persisted when compared to CVT. It achieved a 44.42% reduction in mean QL (relative to CVT's 10070.66) and a 48.69% reduction in median QL, highlighting its strong capability to optimize for central tendency. For the MH metric, the advantage was similarly clear: PM's mean value (366.6) was 9.25% higher than CVT's (335.57). Furthermore, its minimum MH value (319) surpassed CVT's (277) by 15.16%, effectively safeguarding performance under worst-case conditions.

Comparison with D-optimality: The performance advantage of the PM method was most pronounced against the D-optimality design, which performed the poorest in this test, registering a mean QL of 12093.48. The PM method's mean QL was 53.72% lower. For the MH metric, the improvements were substantial, with PM's mean and median values exceeding D-optimality's by 14.78% and 15.32%, respectively. D-optimality's minimum MH value (252) was among the lowest, whereas PM's minimum of 319 was 26.59% higher, demonstrating a decisive advantage in preventing severe performance degradation.

Comparison with G-optimality: Against the G-optimality design, the PM method reduced the mean and median QL by 41.62% and 41.11%, respectively (from G-optimality's mean of 9588.88). The improvements in the MH metric were also considerable, with the mean and median values being 9.57% and 8.92% higher. G-optimality yielded the lowest minimum MH value (235) of all methods; in contrast, PM's minimum of 319 was 35.74% higher. This demonstrates that the PM method not only achieves higher average performance but also establishes a robust performance floor, ensuring system-wide robustness.